Effectiveness of Dynamic Speed Feedback Signs

Volume I: Literature Review and Meta-Analysis

Donald L. Fisher

Andrew Breck

Olivia Gillham

Daniel Flynn

May 2017

Volpe National Transportation Systems Center

55 Broadway

Cambridge, MA 02114

# Executive Summary

Excess speed contributes to one-third of all fatal crashes, and crashes cost society hundreds of billions of dollars each year (NHTSA, 2017; NHTSA, 2010). Lowering excess speeds to reduce these human, societal and economic costs is therefore a major focus of safety officials. A variety of tools are available to achieve these speed reductions, including engineering tools such as speed bumps, rumble strips; enforcement tools such as uniformed officer presence or automated speed enforcement; and educational interventions such as social media campaigns and billboards. This study focuses on the Dynamic Speed Feedback Signs (DSFS), which can combine features of all three of these types of tools. As an engineering tools and education by providing drivers with real-time feedback on vehicle speed, and as an enforcement tool DSFS can be combined with automated enforcement. This study presents evidence showing that DSFS can be effective in a wide range of contexts, with consistent support from studies. Across all types of vehicles and different installation locations, the vast majority of studies found significant reductions in speeds. Overall, reductions of 4 miles per hour (mph) were detected as a result of DSFS installation for passenger cars, and reductions between 2-4 mph detected across vehicle types in the different contexts assessed. As reductions in speed of 5% can reduce fatal accidents by 20% (Nilsson, 2004), these effects demonstrate that DSFS can be effective tools in saving lives.

Three levels of analysis are presented in this study: a Literature Review (Section 2, this Volume) discusses the number of studies with significant findings across the 43 publications reviewed. The Literture Review also discusses the multiple hypotheses that authors have used in their studies, and presents a unified framework to consider all of these hypotheses. The Literature Review assesses the number of significant findings for reductions in mean speed, the 85th percentile of vehicle speeds, and proportion of vehicles over the speed limit. A Meta-Analysis (Section 3, this Volume) then re-analyzes the data from the published literature to present the size of the mean speed reduction effect, accounting for different contexts, vehicle types, and methods of measuring DSFS effectiveness. The statistical approach of the meta-analysis requires detailed data on the number of vehicles sampled, mean speeds, and the variability of these speeds. To carry out this analysis, data on 725 sample units was compiled, including over 5,000 data points. Finally, an Annotated Bibliography (Volume II) presents the details of each of the 43 publications reviewed in a consistent format, allowing detailed examination of the sign types, study designs, and unique characteristics of each study.

This study addresses the effectiveness of DSFS at several contexts. The contexts, referred to as safety foci, present cases where highway engineers and safety officials sought to reduce driver speed in advance of different types of changes in roadway conditions. The main safety foci considered are work zones, school zones, horizontal curves, and transition zones from rural to settled areas.

In work zones, drivers are required to increase their attention not only to address the reduced speed of other vehicles, but also to address added dangers of construction equipment, roadway design and markings, and pedestrian activity. The percentage of fatal crashes in work zones is which are speeding related is similar to the overall value of 27% (NHTSA 2017). The review of the literature found that 45 of 52 studies showed a significant decrease in speeds in work zones when DSFS were installed, with speed reductions of 2.75 mph being detected overall.

School zones showed a similar effectiveness for DSFS, with 24 of 28 studies showed significant reductions in mean vehicle speed during sign activation. Speed reductions of 3.21 mph were detected overall in school zones across all vehicle types, during DSFS activation.

Rural roads are the most dangerous roadway function class, in terms of speeding-related fatal crashes, with nearly half of all speeding-related fatal crashes in 2015 (47%, NHTSA 2017) occurring on rural non-Interstate roads. As drivers transition from rural roads to settled areas, the required reductions in speed may be substantial. DSFS installations were effective in significantly reducing vehicle speeds in all 29 studies which examined DSFS activation effects.

Horizontal curves require full attention of drivers, with studies showing that longer reaction times are required to maintain safe vehicle operation on curves. DSFS are effective in reducing speeds in this context, with all 29 studies which assessed mean speeds presenting significant reductions in speed. The Meta-Analysis found speed reductions were 2.27 mph overall along curves.

In the Meta-Analysis (Section 3), vehicle type considered as well as safety focus, and was found to be an important consideration in measuring DSFS effectiveness. Across all of the hypotheses and safety foci considered, passenger cars demonstrated larger reductions in mean vehicle speed than trucks. Considering the effect of DSFS activation (see below), the size of the effect for passenger cars was approximately double for passenger cars (4 mph), compared to trucks (2 mph). These effects of safety foci and vehicle type were considered for each of method of assessing DSFS effectiveness.

In deciding if and in what situation to install a DSFS, traffic engineers must consider what type of speed reduction effect is desired. As there are different types of outcomes possible, measuring the effect of DSFS installation requires considering the desired outcome and what the baseline comparison case should be. This study considers three types of effects that DSFS can have on vehicle speeds: the Activation Effect, the Downstream Effect, and the Deactivation Effect. Published studies consider different combinations of these effects by different names, and this study combines them all into a unified framework for the first time.

Figure 1. Activation Hypothesis, same site

The Activation Effect is most directly measured as the change in speed of a vehicle at a given site, comparing speeds prior to installation of a DSFS to speeds when the sign is installed and active (Figure 1). In the case where a DSFS is intended to reduce driver speeds immediately at a specific location, the Activation Effect is the appropriate effect to focus on. This is direct measure is commonly made in the literature, and is adequate for a considering in general how much a DSFS can reduce driver speed. One consideration, however, is that roadway conditions may have changed between the two time points that are measured, so that vehicle speeds may differ for reasons other than the DSFS installation.

Figure 2. Activation Hypothesis, same time

Therefore, an alternative way that authors have addressed the Activation Hypothesis is by measuring speeds at one time point, comparing vehicle speeds upstream of the site of DSFS installation to speeds adjacent to the DSFS (Figure 2). As with the “same site” measurement, this method also may be including other reasons why speeds change besides the DSFS. Particularly since DSFS are installed at sites where vehicle speeds are expected to reduce, such as prior to a school zone or along a horizontal curve, this measurement of the Activation Hypothesis may capture changes in speed other than the effect of the DSFS itself. Both of these first two measures of the activation effect have merit, and in the literature both show significant reductions in vehicle speed, but do not fully isolate the effect of the DSFS from other roadway factors.

Figure 3. Activation Hypothesis, normalized

To most accurately capture the effect of DSFS in reducing speed, ideally both aspects of changes in speed at the two sites (upstream and adjacent) and at the two points in time (prior to installation and during installation) would be considered. This normalized measure of the Activation Hypothesis is shows in Figure 3, and accounts for differences in speed which may be due to any factors other than the DSFS itself. This measure of DSFS effectiveness is referred to in this study as a normalized effect, and was encountered less often in the literature, but should be considered the most accurate representation of the activation effect of DSFS in reducing driver speeds.

Figure 4. Deactivation Hypothesis, same time

Safety officials may also consider installing a DSFS upstream of a zone of safety focus. For instance, well upstream of a transition zone between a rural highway and a settled area, it may be desired to begin tapering vehicle speeds. In this case, considering the Downstream Hypothesis would be appropriate. Just as for the Activation Hypothesis, considerations of the baseline comparison case are important, and in the literature both comparisons at the same time (Figure 4) and same site are presented. The Deactivation Hypothesis has more variations than the Activation Hypothesis, since it is also possible to consider comparisons upstream of the DSFS as well. These considerations are discussed further in the Literature Review and Meta-analysis sections of this report.

Figure 5. Deactivation Hypothesis, same site. Removed DSFS indicated in grey.

Finally, safety officials may consider temporarily installing a DSFS, for example as part of a mobile operation in a work zone or as a test case to evaluate DSFS effectiveness. Upon removal of the DSFS, it is possible that a ‘halo’ effect of continued reductions in driver speed may be observed. Some authors have tested this Deactivation Hypothesis. As with the Downstream Hypothesis, there are number of combinations of measurement in time and space which are possible (Figure 5). For deactivation effects as shown in Figure 5, the published literture indicates that these halo effects can occur, although too few studies have been conducted to assess the statistical significance of this effect.

These different hypotheses represent different goals that highway engineers and safety officials may have in reducing driver speeds. The published literature includes all of these variations, and this study presents these hypotheses in a unified framework. By reviewing results and re-analyzing the data from the published literature, this study shows that DSFS are effective in reducing vehicle speeds across all cases. The speed reductions of 4 mph for passenger cars can lead to substantial reductions in speeding-related crashes, reducing the human toll of fatal crashes.

Table of Contents

1.	Executive Summary	2

2.	Literature Review	6

2.1.	Summary	6

2.2.	Introduction	8

2.3.	Literature Search Methods	10

2.4.	Types of Dynamic Feedback Signs	13

2.5.	Article Review Template	14

2.6.	Vote-count Summary	14

2.7.	Hypotheses	17

2.8.	Dependent Variables	23

2.9.	DSFS Implementation Characteristics	24

2.10.	Study Design Characteristics	28

2.11.	Vote-Count Summary by Safety Focus	32

2.12.	Discussion	49

2.13.	References	51

3.	Meta-Analysis	55

3.1.	Summary	55

3.2.	Introduction	55

3.3.	Methods	55

3.4.	Results	59

3.5.	Bibliography	69

# Literature Review

## Summary

Speed has long been known to be a major contributor to crashes. This study addresses the effect that dynamic speed feedback signs (DSFS) can have on lowering speeds, on behalf of The National Highway Traffic Safety Administration and the Governor’s Highway Safety Association. An initial review of the literature indicated DSFS can reduce both mean speed and the 85th percentile speed. Accordingly, the Volpe National Transportation Systems Center undertook a complete search for all studies of DSFS. For each of the domestic studies which met the initial relevance and quality screening criteria, a detailed analysis of the study was undertaken, and a review was written; these reviews are included in the companion Annotated Bibliography. This report contains a description of the literature review approach, including the characteristics of the studies and method for selecting and reviewing them, and a summary of the conclusions.  The section is organized into four major areas:

Literature Search.  First, the report contains a description of the procedures that were used to identify the corpus of studies that were included in the initial count. A total of 108 national and international studies were identified.  It was decided to focus on just the domestic studies.  A total of 77 domestic studies were reviewed, of which 43 passed the relevance and quality screening.

Overall Summary of the Effects of DSFS.  The literature review contains an overall summary of the results known as a vote-count, which summarizes the number of analyses of DSFSs showing significant or non-significant effects for each hypothesis: the DSFS activation hypothesis (labelled here as H1), the downstream activation hypothesis (H2), and the DSFS deactivation hypothesis (H3).  Vote-counts are given for the change in the mean speed, the 85th percentile speed, and the percentage of drivers traveling over the maximum posted speed limit for each of the three hypotheses.

With respect to the change in the mean speed at the DSFS when it was activated (H1), 145 separate analyses were reported.  Of these, 133 were consistent with decreases in speed at the DSFS when it was activated; eight showed no change; and four showed an increase.  With respect to the change in the mean speed downstream of the DSFS when the DSFS was activated (H2), 88 separate analyses were reported.  Sixty of these were consistent with decreases in the mean speed downstream of the DSFS, 22 showed no change, and six showed an increase in speed.  Finally, with respect to the change in the mean speed after the DSFS was deactivated (H3), a total of three analyses were reported.  All showed a decrease in the mean speed at the DSFS.

A Typology of Hypotheses, Dependent Variables, Implementation Variables, Experimental Design.  Third, the literature review includes a detailed discussion of the hypotheses that were evaluated, the dependent variables that were used to evaluate these hypotheses, the characteristics of the implementation of a DSFS that provide traffic engineers with the information that they need for planning a successful DSFS installation, and the details of the experimental design that can provide researchers with the information that they require in order to determine exactly how the effects of the DSFS were evaluated.  This report is the first systematic synthesis of the numerous hypotheses, combination of dependent and implementation variables, and experimental design characteristics. We believe that the typology of hypotheses itself is an important outgrowth of the literature review and a key ingredient in future literature reviews.

Safety Focus Summary.  Finally, the report includes a condensed review of the articles, separating out for each safety focus (work zone, transition zone, horizontal curve, school zone, other) the number of studies consistent with the three hypotheses.  Critically, the safety focus summary includes a discussion of why those studies which were not consistent with the hypotheses might have included elements in the implementation of a DSFS that are obvious things to avoid when implementing a DSFS.

## Introduction

### Background

Speeding-related crashes continue to be a serious problem. Over the last decade, nearly one-third (27% in 2015) of all fatal crashes have been speeding-related (National Highway Traffic Safety Administration, 2017).  NHTSA’s 2011 National Survey of Speeding Attitudes and Behaviors found that 30% of drivers were regular speeders and another 40% of drivers sped at least some of the time (Schroeder, Kostynuik, & Mack, 2013).  In this same survey, the most widely approved countermeasure for speeding was dynamic speed feedback signs (DSFSs).  The use of these signs was supported by 87% of speeders and 91% of sometimes speeders.  Similarly, focus groups in NHTSA’s Motivations for Speeding study were also very supportive of dynamic speed feedback signs (Richard, et al., 2013).

There have been many studies focusing on the effectiveness of dynamic speed feedback signs (DSFS), also known as radar activated speed feedback signs, in reducing vehicle speeds. These studies vary substantially in roadway context, design, and study goal. An initial scan of the literature by NHTSA indicated that the studies focusing on DSFSs have included conditions with varying safety foci (roadway departures, pedestrians, intersections, work zones), roadway types (collector, arterial, interstate, local), speed limits (from 25 to 65 mph), DSFS characteristics (e.g., stationary versus mobile units), and locations (urban, suburban, rural).  There was at least one study that looked at the use of dynamic speed feedback signs in conjunction with speed enforcement (Bloch & Automobile Club of Southern California, 2007). There was also at least one literature synthesis (Donnell & Cruzado, 2007) which covered 12 research studies on this countermeasure. The general findings of these studies showed reductions in both mean speeds and 85th percentile speeds while the devices are deployed.  Finally, there was at least one study (Tribbett, McGowen, & Mounce, 2000) that showed a reduction in crashes.

### Objective

Given the large number of previous studies, end users in the highway safety community are faced with the challenge of working through the existing studies to locate appropriate research that addresses their needs.  In order to reduce that burden, the Volpe National Transportation Systems Center (Volpe) was asked to conduct a comprehensive review of the existing literature on dynamic speed feedback signs from 2000 to the present, produce an Annotated Bibliography of all studies found, and undertake a systematic and quantitative review of these studies, including  a meta-analysis.  This report aims provide an important resource for both highway safety practitioners interested in reducing speeding and speeding-related crashes and researchers interested in doing further research on this important and widely accepted traffic safety countermeasure.

We begin this report with a summary of the literature review findings, and follow with a general discussion of dynamic speed feedback signs so that readers are familiar with the different types of dynamic speed feedback signs that have been used.  We then continue with a discussion of:

- the methods used in the literature search,

- the overall, systematic, quantitative review (the vote-count),

- the typology of the characteristics of the studies including:

  - the three different hypotheses that were evaluated,

  - the three dependent variables that were used to evaluate those hypotheses,

  - the variables that define a DSFS implementation, and

  - the different experimental designs that were used to evaluate the hypotheses, and

- the more detailed report (vote-count) of the effectiveness of DSFSs at each of the five different safety foci for each of the three different hypotheses and three different dependent variables along with an in depth discussion of those studies which showed no effect of the DSFS on the mean speed at the DSFS or an actual increase in speed at the DSFS after activation of the DSFS.

The report provides a quantitative summary of the findings of the literature review on the effects of DSFSs, both overall and by safety focus.  The quantitative summary is typically referred to as a vote-count, in the sense that simply the number of statistically significant results are tallied; this approach provides the foundation for the formal Meta-Analysis, which directly analyzes the underlying data from each study in a common quantitative analysis (Section 3). This review considers three main tests of the efficacy of DSFS: whether activation of the DSFS reduces some function of speed at the DSFS (referred to as Hypothesis 1 or H1), whether the activation of the DSFS reduces some function of speed downstream of the DSFS (H2), and whether deactivation of the DSFS leads to increases in some function of speed at the DSFS after the DSFS is deactivated (H3). Three functions of speed were considered: the mean speed, the 85th percentile speed, and the percentage of drivers over the speed limit. For each of the five safety foci (work zone, school zone, transition zone, curved section, straight section), not only is a vote-count provided, but an analysis is made of each of the studies where the mean speed at the DSFS was either not changed or actually increased.

### Summary of Findings

This review of the literature demonstrates that driver speeds are reduced by the installation of DSFS. By summarizing the number of studies which find significant reductions in speed under different circumstances (a vote-counting review), we show that speeds are reduced under all three of the main hypotheses: Speeds are reduced when a DSFS is activated, at the location of the sign (H1); speeds are reduced downstream of the DSFS when the sign is activated (H2), and speeds are reduced at the DSFS after the sign has been deactivated.

In the evaluation of the hypothesis of a speed reduction at the location of the sign when active (H1), 133 sites showed statistically significant reductions in the mean speed, while eight sites presented no change in the mean speed, and four sites found a statistically significant increase in the mean speed. The support for the effect of the DSFS on speeds downstream of the DSFS (H2) is less strong than support for the effect at the DSFS (H1). Fully 92% of the analyses show a significant reduction in speed at the DSFS whereas only 68% of the analyses show a significant reduction in speed downstream of the DSFS. DSFSs clearly reduce driver speeds.  There appears to be a halo effect after the DSFS is deactivated, although the sample size is limited.

Using a vote-count, not only is there support for the effectiveness of DSFSs on reductions in the mean speed, but there is also support for the effectiveness of the DSFSs on reductions in the  85th percentile speed and the percentage of drivers traveling above the speed limit at the DSFS and downstream of the DSFS in large numbers.

The vote-count shows only the number of studies that provide evidence consistent with or not consistent with a given hypothesis.  One would also like to know the exact size of the reduction in speed in order to predict the effect on crashes (Tefft, 2011).  The formal meta-analysis will show the quantitative reductions in speed to be expected under different circumstances, for each of the hypotheses tested.

## Literature Search Methods

The search for documents which report the effect of DSFSs on drivers’ behavior was undertaken between March 9, 2016 and March 16, 2016 by Volpe and MIT library staff. A total of 108 national and international studies were identified.  It was decided to focus on just the domestic studies.  A total of 77 domestic studies were reviewed, of which 43 passed the relevance and quality screening.

### Search Words and Phrases

The words and phrases used in the literature search were relevant to the project’s key research questions:

- How are DSFSs being used to regulate speed?

- What are the extant findings regarding the effectiveness of DSFSs?

Searches started with the keywords (including the word variations) listed in Group 1 in the box below (Table 1). If the number of items retrieved was too large with the keywords used from Group 1, then the search words from Group 1 were combined one at a time with the keywords listed in Group 2.  Only one of the Group 1 terms met these criteria, “speed limit signs”, with over 2660 records.  Any of the search terms in the tables that ended in ‘s’ were wildcards with *. Wildcards allow one to search for all variants of a term.  For example, “warning sign*”, includes “warning sign”, “warning signs” and “warning signage”.

Table 1.  Search Words

| Group 1 | Group 2 |
| --- | --- |
| - Speed - Dynamic speed display signs (DSDS) - Speed display signs - Dynamic warning signs - Dynamic speed feedback signs (DSFS) - Speed feedback signs - Driver feedback signs - Speed monitoring displays (SMD) - Speed displays - Speed display trailer - Dynamic displays - Speed warning signs - Variable message speed limit signs - Speed limit signs - Speed minders - Speed indicator device (SID) - Speed reduction treatments - Radar speed signs - Radar speed check signs - Photo-radar displays - Curves - Dynamic curve warning signs - Advance curve warnings - Traffic calming measures | - Locations - Curves - School zones - Work zones - Intersections - Transition zones - Treatment effects - Speed reduction - Speed decreases - 85th percentile speed reductions |

Seven extensive databases were searched: Transportation Research International Documentation (TRID), National Transportation Library, WorldCat, Academic Search Complete, PsychINFO, Web of Science, and Science Direct. Together, these databases provide a comprehensive coverage of academic and governmental on transportation-related research.

### Results of Search

The search yielded a total of 108 references, placed into different categories (Table 2).  It was decided initially to review only domestic studies given the differences in roadway design, traffic regulations, and driving cultures around the world.  This led to the exclusion of 31 studies. Of the 77 remaining domestic sources, 18 were redundant.  For example, a study which appeared as a proceedings article with one set of authors and title might appear as a journal article with different authors, different titles, or seemingly different content.  Of the remaining 59 articles, three could not be located (retrieved from the URL or other internet searches), leaving 56 articles to be screened for relevance and quality.

Table 2.  Summary of publications evaluated for literature review.

| Decision | Number |
| --- | --- |
| Included | 43 |
| Data article (42) |  |
| Review article (1) |  |
| Redundant | 18 |
| International | 31 |
| No speed data | 6 |
| Sign not relevant to speeds | 6 |
| Not able to retrieve | 3 |
| Not reviewed | 1 |
| Synthesis (included) | 1 |
| Total | 108 |

#### Study Identifying Information

We begin each review of an article by recording the relevant identifying information (example shown in Table 3).  The authors and date can be used to retrieve the full reference in the Annotated Bibliography and in this document.

Table 3.  Literature Review: Example Study Identifying Information

| Study Characteristics | Response |
| --- | --- |
| - Authors | Bullough, Skinner, Brons, and Rea |
| - Title | Using Lighting and Visual Information to Alter Driver Behavior |
| - Source | Volpe |
| - Publication year | 2012 |

#### Relevance Screening

The screening process was two-tiered.  First, a study was reviewed for relevance.  If it passed this screening, the study was then reviewed for quality.  To measure study relevance, Volpe used the relevance screening tool in Table 4, adapted from a leading handbook (Cooper, Hedges, & Valentine, 2009).  To use this tool, Volpe filled in a table for each study by answering questions 1 and 2.  If Volpe answered “No” to one or both of questions 1 and 2, Volpe excluded that study from the literature review.

Table 4.  Literature Review: Relevance Screening

| Inclusion Criteria | Response (Yes or No) |
| --- | --- |
| - Is this study an empirical investigation of the effects of effectiveness of dynamic speed feedback signs? | Yes |
| - Is the outcome measure crashes, fatal crashes, and/or some function of traffic speeds? | Yes |

In six of the studies, the signs that were used did not report the speed of the driver or, if it reported the speed, did not indicate whether the driver was above the speed.  That left 49 studies to review for quality.

#### Quality Screening

To measure the quality of the studies that made it through the relevance screening, Volpe used the quality screening tool shown in Table 5, adapted from Cooper, Hedges, and Valentine (2009). Similar to the relevance screening tool procedure, Volpe filled in one table for each study by answering questions 1 through 4.  Volpe then assessed which studies were appropriate for inclusion in the literature review.

Table 5.  Literature Review: Quality Screening

| Quality Criteria | Response (Yes or No) |
| --- | --- |
| - Were the study population and the context of the study well described? | Yes |
| - Were the exposure variables valid implementations of the conditions that they were meant to represent? | Yes |
| - Was the outcome variable both a reliable and valid measure of the outcome of interest? | Yes |
| - Does the study contain sufficient data of an appropriate quality to be used in the meta-analysis? | Yes |

In six of the studies no speed data were reported.  That left a total of 43 studies, one of which is a literature review.  The literature review was not included in the set of 42 studies that were used for the vote-count.

## Types of Dynamic Feedback Signs

Across these studies, a number of different types of DSFS were employed. For purposes of this report, they include portable changeable message signs (PCMS), speed monitoring displays (SMD), and speed display trailers (SDT), just to mention a few.  The configurations of signs and messages which are included as part of a DFSF deployment vary from one installation to another. Examples of the types of configurations commonly encountered are presented in Appendix B, and details on the configurations of signs in each study are presented in the Annotated Bibliography in Volume II.

## Article Review Template

Each of the 43 included articles was reviewed in detail and will be included in a separate Annotated Bibliography.  Each review consists of the following four sections:

- A list of the features of the study;

- A summary of the study;

- A schematic indicating the where the sensors and DSFS were located as well as the geometry of the roadway where the treatment was applied;

- A graphic of the DSFS (or DSFSs) used in the study.

A detailed description of each of the above four sections is included in Appendix C of Volume II.

## Vote-count Summary

Before delving into the details of the specific formulation of the thirteen different hypotheses, the dependent variables, the characteristics of a particular implementation of a DSFS, and the specifics of the experimental design, we want to describe the overall results in a way that is comprehensive, quick to understand, and representative of the much more detailed analyses we present below and in a related meta-analysis.  Briefly for each general hypothesis and each dependent variable that we analyzed, we counted the number of sites where a decrease, an increase, or no change in speed was noted.  We then present these summaries as what is known as a vote-count (Koricheva & Gurevitch, 2016). In a vote-count, the number of significant results for each alternative hypothesis are simply summed up across studies. This type of review is quantitative in the sense that significant results are summarized, but does not take into account the size of the effect (i.e., how many miles per hour were reduced as a result of the DSFS), the sample size, or the variability of the effect size. These factors require more data to consider, and are included in the meta-analysis in Section 3.

In order to present the vote-count, we first need to define briefly the hypotheses that we evaluated and the dependent variables that we used to evaluate them.  We then go on to present the vote-count.  The hypotheses are defined in more detail in Section 2.7, the dependent variables in Section 2.8.

### Hypotheses

Ultimately, we want to know for each study whether the DSFS reduces the speed of the drivers.  Three key questions can be addressed when considering DSFS effectiveness:

- Does the activation of the DSFS reduce speed at the DSFS?

- Does the activation of the DSFS reduce speed downstream of the DSFS, and

- Do speeds continued to be reduced at the DSFS after the DSFS was deactivated?

With this in mind, we have defined three general hypotheses:

- H1 – the Activation Hypothesis,

- H2 – the Downstream Hypothesis, and,

- H3 – the Deactivation Hypothesis.

### Dependent Variables

We decided to focus on three different dependent measures of effectiveness for each of the three hypotheses:  the mean speed, the 85th percentile speed, and the percentage of drivers traveling faster than the posted speed limit.

### Systematic, Quantitative Review: Vote-counts

The evidence from the vote-count is consistent with the conclusion that speeds are reduced when a DSFS is activated at the DSFS (H1), are reduced downstream of the DSFS when the DSFS is activated (H2), and are reduced at the DSFS after the DSFS is activated.  As just one example, consider the effect of the activation of the DSFS on the speed at the DSFS (H1).  We found 133 sites where a statistically significant reduction in speed was recorded, eight sites where no change in speed was recorded, and four sites where a statistically significant increase in speed was recorded.

We begin with a discussion of how the vote-count was conducted and then provide that count for each of the three hypotheses and each of the dependent variables.

#### Vote-count Procedure

The procedure went as follows.  Assume that a study had n sites with one statistical evaluation at each site.  Suppose that for a particular hypothesis and particular dependent variable, the results from m of the n sites were consistent with a particular hypothesis, m < n.  Then, m votes were tallied as consistent with the hypothesis and n – m votes as not consistent with the hypothesis. The n – m were further partitioned into those that showed no change in speed and those that showed an increase in speed.

Consider a single hypothetical study. Suppose that the mean speed was measured at the DSFS before activation and then again at the DSFS during activation (H1B, a particular variation of the DSFS activation hypothesis).  Suppose that there were 12 sites.  Suppose that significant reductions in speed were recorded at 9 of the 12 sites, no significance change in speeds were recorded at two of the sites, and an increase in speed was recorded at one site.  Then, Volpe would add nine votes as supporting the DSFS activation hypothesis (a reduction in speed at the DSFS when it is activated), two votes as showing no change in speed, and one vote as showing an increase in speed.

In the unusual case that several vehicle classes were measured at one site, these were counted separately. So, for example suppose there were two sites and the speed of both trucks and cars were recorded at both sites.  Then if the speeds were reduced at the DSFS for cars and there was no change for trucks, then this would be recorded as two votes for a decrease and two votes for no change.

#### H1: Activation Hypothesis

The vote-count for the DSFS activation hypothesis, aggregated across all three variations in this hypothesis (see Section 2.7), is presented in

Figure 6 separately for the mean speed, the 85th percentile speed, and the percentage of drivers over the speed limit.  As already noted above, out of 145 separate statistical evaluations of the decrease in speed at the DSFS, fully 133 showed a significant decrease, eight showed no change, and four showed an actual increase.  In the break out of the vote-count by safety focus we discuss factors potentially leading to no change or an actual increase in speed for each safety focus.  Here we simply note that the overwhelming majority of studies, 92%, were consistent with the hypothesis that the speed would decrease at the DSFS.  Although there were fewer analyses of changes in the 85th percentile speed and changes in the percentage of drivers traveling over the speed limit, in both cases the number of studies consistent with the hypothesis that there would be a reduction in the 85th percentile speed and the percentage of drivers traveling over the speed limit was clearly much larger than the alternative hypothesis.  With respect to reductions in the 85th percentile speed, fully 116 of the 121 separate statistical analyses showed a significant reduction (95.9%).  And with respect to reductions in the percentage of drivers traveling over the speed limit, 61 of the 65 statistical analyses showed a significant reduction in this percentage (93.8%).

Figure 6. H1: Activation Hypothesis

#### H2: Downstream Activation Hypothesis

The change in speed downstream of the DSFS while the DSFS is activated can be measured with respect to the speed upstream of the DSFS or the speed at the DSFS.   The vote-count for the downstream activation hypothesis (H2) is based on the upstream speed as the reference (H2A, H2B and H2C; see Section 2.7.2) and is presented in Figure 7. Over two-thirds (68.2%) of the studies are consistent with the hypothesis that there would be a reduction in the mean speed downstream of the DSFS during the activation of the DSFS (H2). Compared to changes in speed adjacent to the DSFS (H1), this represents a smaller percentage of studies.  However, the percentage of studies consistent with the hypothesis that there would be a reduction in the 85th percentile speed at the downstream location when the DSFS was activated (92.1%) and that there would be a reduction in the percentage of drivers traveling over the speed limit (88.3%) both remained high.

Figure 7. H2: Downstream Activation Hypothesis

#### H3: Deactivation Hypothesis

There were many fewer studies which reported information relevant to the deactivation hypothesis.  The change in speed at the DSFS after deactivation can be measured relative to the DSFS sensor before activation or relative to the DSFS during activation.  For the studies that measured the speed at the DSFS after deactivation relative to the speed at the DSFS sensor before activation (H3A, H3B, H3C; see Section 2.7.3), all three showed significant decreases in speed.  A decrease would be expected if there was a continuing effect of the activation of the DSFS on the speed of the drivers, even following the removal of the sign.

#### Limitations

The reader should be aware that there are clear limitations to using a vote-count to determining the relative support for or against a particular hypothesis (Koricheva & Gurevitch, 2016).  Among other things, the vote-count does not weight the studies by the number of observations, measure the size of the effect, or show the variability around that effect size.  This information is contained in the final technical report in the section describing the meta-analysis as well as in a separate report to be published as a meta-analysis.

## Hypotheses

As noted above in Section 2.6.1, there are three general hypotheses.  It is worth repeating these general hypotheses at the start of this section.  Briefly, the DSFS activation hypothesis (H1) is used to evaluate the following question:  At the location of the DSFS are speeds changed after the DSFS is activated?  The DSFS downstream activation hypothesis (H2) answers the following question: Downstream of the DSFS, are speeds changed after the DSFS is activated?  The DSFS deactivation hypothesis (H3) answers the question: After removal of the DSFS are speeds changed at the DSFS from what they were when the DSFS was activated? Upon further inspection, it became clear that there were three different variations of H1, five different variations of H2 and five different variations of H3.  Use of this typology would help future researchers by standardizing the terminology, facilitating the search for a study relevant to a hypothesis of particular interest.

### Hypothesis 1: Activation Effect at DSFS

There are three different ways to determine whether there is a reduction in speed at the DSFS.  These different variations are important to consider because they can lead to very different conclusions about the effectiveness of a DSFS.  One might wish it were simply a matter of answering the question: Is the speed reduced or is it not reduced at the DSFS when the DSFS is activated?  However, because studies often used different ways of answering this question, and because there are different ways of evaluating the hypothesis, it is important that the reader come to some understanding of how the different variations of the DSFS activation hypothesis are formulated.

To assess how the activation of a DSFS reduces driver speed, the ideal measurement would account the change in vehicle speed from a location upstream of the sign to the site of the sign. Since this difference differences in the upstream speed across time.  We get the quantity below in Equation :

H1A: DuringDSFS-BeforeDSFS-

DuringUpstream-BeforeUpstream

Equation 1

In Equation  the difference of differences is now negative if the reduction in speed observed between measurements made at the DSFS before and during activation is larger than the change in speed observed upstream before and during activation.

More simply, one can compare the speed at the DSFS during activation to the speed at the DSFS before activation.  We can represent this quantitatively as the difference in the speed at the DSFS during the activation of the DSFS (DuringDSFS ) and the speed at the DSFS before the activation of the DSFS ( BeforeDSFS):

H1B: DuringDSFS-BeforeDSFS

Equation 1

We refer to this as Hypothesis H1B (measures made at same location at different times).  This is made clear graphically in Figure 8.

Note that the colors of the measurements (blue in the above example) come from the figure.  Locations of the sensor measuring speed (upstream of the DSFS, adjacent to the DSFS, or downstream of the DSFS) appear down the rows.  The times during which the measurements of speed are made (before activation of the DSFS, during activation of the DSFS, or after deactivation of the DSFS) appear across the columns. Note the result in Equation 1 is negative if

there is a reduction in speed.

| Hypothesis Code | Hypotheses | Speed reduction formula | Required Data |
| --- | --- | --- | --- |
| H1A | Activation Effect at DSFS (Normalized) | DuringDSFS-BeforeDSFS- DuringUpstream-BeforeUpstream | Timing. Before; During; After. Location; Upstream. Adjacent. Downstream. |
| H1B | Activation Effect at DSFS (Same Site) | DuringDSFS-BeforeDSFS | Timing. Before; During; After. Location; Upstream. Adjacent. Downstream. |
| H1C | Activation Effect at DSFS (Same Time) | DuringDSFS-DuringUpstream | Timing. Before; During; After. Location; Upstream. Adjacent. Downstream. |

Figure 8.  Hypotheses H1A, H1B and H1C.

Second, one can compare the speed at the sensor upstream of the DSFS during activation of the DSFS with the speed at the DSFS during the activation of the DSFS (H1C; measures made at different locations at the same time).  This appears in Figure 8 and is expressed quantitatively as:

H1C: DuringDSFS-DuringUpstream

Equation 2

Again, the difference is negative if there is a reduction in speed.

Either of the above two comparisons has problems.  H1B does not control for differences in the upstream speed before and during activation.  H1C does not control for differences in the speed before activation that may exist between the sensor upstream of the DSFS and at the DSFS.  Algebraically, controlling for the differences in H1B leads to a quantity which is identical to what is obtained when one controls for the differences in H1C; either way results in H1A.

A simple example can show why these different ways of computing the activation effect of the DSFS are critical.  Assume the following values for the speeds measured before and after the activation of the DSFS both upstream of the DSFS and adjacent to the DSFS:

- DuringDSFS=30

- BeforeDSFS=40

- DuringUpstream=45

- BeforeUpstream=50

Then, evaluating H1A we find a reduction in speed of 5 mph (Figure 9).  Evaluating H1B we find a reduction in speed of 10 mph.  And evaluating H1C we find a reduction in speed of 15 mph.  It is important to repeat that any negative outcome is consistent with a favorable outcome, i.e., a reduction in some function of speed when the DSFS is activated.  Everything else being equal, the different ways of evaluating H1 could lead to large differences in the effect sizes computed in a meta-analysis.

| Hypothesis Code | Hypotheses | Speed reduction formula | Required Data |
| --- | --- | --- | --- |
| H1A | Activation Effect at DSFS (Normalized) | DuringDSFS-BeforeDSFS- DuringUpstream-BeforeUpstream =-5 | Timing. Before; During; After. Location; Upstream; 50; 45. Adjacent; 40; 30. Downstream. |
| H1B | Activation Effect at DSFS (Same Site) | DuringDSFS-BeforeDSFS=-10 | Timing. Before; During; After. Location; Upstream. Adjacent; 40; 30. Downstream. |
| H1C | Activation Effect at DSFS (Same Time) | DuringDSFS-DuringUpstream=-15 | Timing. Before; During; After. Location; Upstream; 45. Adjacent; 30. Downstream. |

Figure 9. Quantitative Example: Hypotheses H1A, H1B and H1C.

### Hypothesis 2: Downstream Activation Effect

If one wants to know whether there is a reduction in speed downstream of the DSFS after the DSFS has been activated, unlike with Hypothesis 1 where there where there were two possible pairwise comparisons (H1B and H1C), there are now three possible comparisons: the downstream sensor before activation with the downstream sensor during activation; the upstream sensor during activation with the downstream sensor during activation, and the DSFS sensor during activation with the downstream sensor during activation.

Consider the first two comparisons: the speed at the downstream sensor before activation with the speed at the downstream sensor after activation (labelled H2B) and the speed at the upstream sensor during activation with the speed at the downstream sensor during activation (H2C).  If we subtract the comparison sensor speed from the downstream sensor speed during activation the result should be negative. Moreover, when we control either for changes in speed that might have occurred simply as a function of differences in the time of the two measurements that are used to evaluate H2B or for the changes in speed that occurred simply as a function of differences in the locations of the two measurements that are used to evaluate H2C, we get one and the same algebraic quantity (which we label as H2A, unprimed). These three hypotheses and their corresponding formulas are represented in Figure 10 (H2A, H2B, and H2C, parallels of H1A, H1B and H1C).  It is clear from the computations why, as before, it is important to be certain about just what hypothesis one is evaluating.  Moreover, as before, any computation which involves a significant negative result is associated with a favorable outcome.

| Hypo-thesis Code | Hypotheses | Speed reduction formula | Required Data |
| --- | --- | --- | --- |
| H2A | Activation Effect Downstream of DSFS (Normalized) | DuringDownstream-BeforeDownstream- DuringUpstream-BeforeUpstream= -5 | Timing. Before; During; After. Location; Upstream; 50; 45. Adjacent. Downstream; 35; 25. |
| H2B | Activation Effect Downstream of DSFS (Same Site) | DuringDownstream-BeforeDownstream=-10 | Timing. Before; During; After. Location; Upstream. Adjacent. Downstream; 35; 25. |
| H2C | Activation Effect Downstream of DSFS (Same Time) | DuringDownstream-DuringUpstream=-20 | Timing. Before; During; After. Location; Upstream; 45. Adjacent. Downstream; 25. |
| H2A' | Activation Effect Downstream of DSFS (Normalized) | DuringDownstream-DuringDSFS- BeforeDownstream-BeforeDSFS=0 | Timing. Before; During; After. Location; Upstream. Adjacent; 40; 30. Downstream; 35; 25. |
| H2C' | Activation Effect Downstream of DSFS (Same Time) | DuringDownstream-DuringDSFS=-5 | Timing. Before; During; After. Location; Upstream. Adjacent; 30. Downstream; 25. |

Figure 10. Quantitative Example: Hypotheses H2A, H2B, H2C, H2A and H2C.

Finally, consider the third comparison, the speed at the adjacent sensor during activation with the speed at the downstream sensor during activation (H2C). The desirable outcome is no change in the speed at the downstream sensor, the hope being that the effect of the DSFS will not dissipate with distance from the DSFS.   If we control for the differences in speed that might occur simply because of a difference in location, we get the normalized quantity we label as H2A.

Note that we can normalize H2B either with respect to the changes in speed across time at the upstream sensors or the changes in speed across time at the adjacent sensors.  The normalized quantity, H2A, for H2B with respect to changes in speed across time at the upstream sensors will not necessarily be the same as the normalized quantity, H2A, for H2B with respect to the changes in speed across time at the adjacent sensors.

An example can help make clear of the difference between H2 unprimed and H2 primed hypotheses (Figure 10).  Note that the results are all negative for the three variations of H2: H2A, H2B and H2C.  If the result were negative and significant, the result is consistent with the hypothesis that there is an effect of the activation of the DSFS on the downstream sensor.  However, look now at the results for H2 primed where the comparison point is adjacent to the DSFS.   If we look only at the difference in the speed of the vehicles at the DSFS and the downstream sensor while the DSFS is activated, then we see a reduction of 5 mph (H2C).  If we look only at the difference in speed at the downstream sensors across time we see a reduction in speed of 10 mph (H2B or H2B).  However, consider the normalized versions of H2B and H2C, what we refer to as H2A.  Here, we find no change in speed.

Finally, we should note that, as discussed above, the normalization of H2B with respect to the upstream sensors shows a reduction in speed of 4 mph (H2A = -5) whereas the normalization of H2B with respect to the adjacent sensors shows no change in speed (H2A = 0).  Both answers are correct as normalizations of H2B (a reduction in speed of 5 mph, H2A, or no reduction in speed, H2A), they just reference different questions or hypotheses.

### Hypothesis 3: Deactivation Effect at the DSFS

Just as there are three pairwise comparison for the downstream sensor, so to there are three pairwise comparisons for evaluating the effect of the deactivation of the DSFS on speed at the DSFS: the speed at the DSFS after its deactivation with the speed at the DSFS before its activation; the speed at the DSFS after its deactivation with the speed adjacent to the DSFS after its deactivation; and the speed at the DSFS during the activation to the speed at the DSFS after its activation.  Note that as with Hypothesis 2, with Hypothesis 3 the first two pairwise comparisons should be negative if the speed at the DSFS after its deactivation is slower than it was before its activation.  Similarly, the third pairwise comparison should show no change if the speed at the DSFS after its deactivation does not increase from what the speed was at the DSFS during its activation.  Again, there are two unprimed and three primed different ways of making the computations with the un-normalized (H23, H2C, H2B´) and normalized (H3A, H3A´) hypotheses (Figure 11). And as before, one of the primed ways (H3C) is redundant with one of the unprimed ways (H3C) and so is not listed in the primed set.

| Hypo-thesis Code | Hypotheses | Speed reduction formula | Required Data |
| --- | --- | --- | --- |
| H3A | Deactivation effect at DSFS (Normalized), relative to before | AfterDFSF-BeforeDSFS- AfterUpstream-BeforeUpstream =-10 | Timing. Before; During; After. Location; Upstream; 50; 55. Adjacent; 40; 35. Downstream. |
| H3B | Deactivation effect at DSFS (Same Site), relative to before | AfterDSFS-BeforeDSFS=-5 | Timing. Before; During; After. Location; Upstream. Adjacent; 40; 35. Downstream. |
| H3C | Deactivation effect at DSFS (Same Time) | AfterDSFS-AfterUpstream=-20 | Timing. Before; During; After. Location; Upstream; 55. Adjacent; 35. Downstream. |
| H3A' | Deactivation effect at DSFS (Normalized), relative to during | AfterDSFS-DuringDSFS )-(AfterUpstream- DuringUpstream=-5 | Timing. Before; During; After. Location; Upstream; 45; 55. Adjacent; 30; 35. Downstream. |
| H3B' | Deactivation effect at DSFS (Same Site), relative to during | AfterDSFS-DuringDSFS=5 | Timing. Before; During; After. Location; Upstream. Adjacent; 30; 35. Downstream. |

Figure 11.  Hypothesis H3.

## Dependent Variables

A brief description follows of the dependent variables that were evaluated and the motivation for choosing these dependent variables.

### Description

Three dependent variables were selected: the mean speed, the 85th percentile speed and the percentage of drivers traveling above the speed limit.  The mean speed, 85th percentile speed and percentage of drivers traveling above the speed limit could be evaluated in three separate ways for H1, five ways for H2 and five ways for H3, as described above:

- Mean speed: H1, H2, H3 – report of difference in the mean speed for each hypothesis (column 3, Figure 9, Figure 10, Figure 11) and the statistical significance of the evaluation of the difference in the mean speeds for each hypothesis

- 85th percentile speed: H1, H2, H3 – report of difference in the 85th percentile speed for each hypothesis and the statistical significance of the evaluation of the difference in the 85th percentile speeds for each hypothesis; and

- Percentage of drivers over the speed limit: H1, H2, H3 – report of difference in the percentage of drivers over the speed limit for each hypothesis and the statistical significance of the evaluation of the percentage of drivers over the speed limit for each hypothesis; and

### Motivation

The dependent variables described above, all different functions of speed, are the ones most often reported in the studies and most closely tied to crashes (National Highway Traffic Safety Administration, 2017).  By indicating for each hypothesis and each dependent variable whether there was a significant effect, we are able to provide the overall information on the number of studies showing a significant effect in the overall vote-count analyses above (Section 2.6) and in the vote-count analyses for each separate safety focus below (Section 2.9.1).  Moreover, in the later report on meta-analyses we incorporate as well the effect size and sample size into the resulting analyses.

It is important to keep in mind throughout this report that reductions in speed of even a few miles an hour can have a large impact on fatalities and injuries (Kloeden, McLean, Moore, & Ponte, 1997).   For example, at relatively low speeds the risk of severe injury for 30 year olds decreases from 50% to 25% as the speed at impact decreases by 6 mph from 33 mph to 27 mph (Tefft, 2011).  The risk of severe injury for 70 year olds decreases from 75% to 55% when the speeds at impact are decreased similarly.  At slightly higher speeds, the risk of death for 30 year olds decreases from 50% to 35% as the speed at impact decreases by 4 mph from 44 mph to 40 mph. The risk of death for 70 year olds decreases from 80% to 70% as the speed at impact decreases by the same amount.  The point here is that reductions in speed of even 4-6 mph can be of large practical significance when it comes to reducing the frequency of severe injuries and fatalities.

## DSFS Implementation Characteristics

This section includes a description of the characteristics that define the particular context in which a DSFS was implemented that we believe are important to States.  Additionally, it describes the motivation for the inclusion as well and the particular documents to which one may wish to refer to obtain more information.  Note that this motivation is sometimes based on actual results from published studies.  At other times, the motivation is based on our initial review of the articles and the impression we have formed of what variables may be most important to collect.

### Safety Focus

#### Description

Safety focus: work zone, school zone, transition zone, curve, straight section.  The safety focus refers to the element of the traffic system in which safety is trying to be improved.  There is no standard definition of safety focus.  We identified five different levels (or values) of the safety focus: Work zone, school zone, transition zone, curved section, and straight section.  An example of a transition zone would be the area between the end of a high-speed rural highway and the beginning of a town center.  There are no agreed upon national definitions of some of the safety foci, e.g., work zones (Turner, 2015).

#### Motivation

The reductions in speed that one can expect as a function of the safety focus because of either the geometry (curved versus straight sections) and context (work zone versus school zone) may vary greatly. For example, we have found that the DSFS leads to more dramatic reductions in speed at transition zones than it does in straight sections.

### DSFS (Several Descriptors)

#### Description

We did not find any standard definition of the different features of a DSFS or the values which the features could take on.  Three critical features of the DSFS that could be relevant are below.  In addition, where possible we include in the review of each study in the Annotated Bibliography a graphic of the DSFS that provides readers with an actual picture of what is used.

- Does the DSFS display speed (yes, no)?

- What type of lighting is used for the display of speed (LED, reflective, other)?

- Does the DSFS flash when the speed is exceeded (yes, no)? and,

- Is there a graphic of the DSFS (yes, no)?

When we indicate the DSFS flashes, we are indicating only that something flashes.  It could be the speed, but it could also be some other signal that was activated when the speed was exceeded (e.g., alternately flashing lights).

#### Motivation

There is reason to believe that the effectiveness of the DSFS may vary with the above descriptions.  For example, it is well known that flashing displays attract attention (Christ & Abrams, 2006).  That alone could make a study with a flashing DSFS more effective than one without a flashing DSFS.

### Level of Service

#### Description

Level of service: free flow, other, multiple, not specified.  The level of service is noted as free flow, other or not specified.  There are standard definitions for defining different levels of service (Transportation Research Board, 2010).  Note we use the value “multiple” for a feature when different sites have different levels of service.  More generally, “multiple” will be used throughout to indicate situations in which different sites, or the same site with different treatments, have different values of the feature.

#### Motivation

The activation effect at the DSFS as measured by the difference in speeds between the time before it was activated and the time after it was activated could be due solely to differences in the level of service.  Thus, it is important to know from the standpoint of validating the experimental design.  But, more generally, there may be very different reductions one might expect in free flow conditions than one would expect in congested conditions.  Specifically, in congested conditions one would expect very little effect of a DSFS.  This can be entered into the meta-analysis and will be of interest even in cases where a meta-analysis cannot be performed.

### Roadway Type

#### Description

Roadway type: freeway, arterial, collector, local, multiple, unspecified.  The roadway type feature can assume any one of six different values: freeway, arterial, collector and local, along with multiple types in some studies and unspecified in still other studies.  The Federal Highway Administration defines the classification of roadway networks (Federal Highway Administration, 2012).  Sometimes highway is not differentiated from arterial.

#### Motivation

Roadway type may influence the effectiveness of a DSFS.  It is simply not known, but the information is easy to collect.

### Roadway Area

#### Description

Roadway area: urban, suburban, rural, multiple, unspecified. The roadway area feature can have five different values: urban, suburban, and rural along with multiple and unspecified. The Federal Highway Administration typically uses just the first three classifications (Federal Highway Administration, 2013).  However, some individual studies additionally refer to exurbs.   Urban areas are typically defined as communities with more than 100,000 people.  The population in the suburbs ranges anywhere from 10,000 to over a 1,000,000.  Exurbs are defined as communities with between 1,000 and 20,000 people that are largely inhabited by commuters.  Rural areas are settled places outside towns and cities and usually have fewer than 10,000 people.

#### Motivation

The motivation for including roadway area is the same as the motivation for including roadway type.

### Sidewalks

#### Description

Sidewalk: present, absent, unspecified. A number of articles mention whether sidewalks are present where the DSFS is positioned.  The feature has three values: present, absent or not specified.

#### Motivation

It is arguably the case the sidewalks should have a speed calming effect on drivers (South Carolina Department of Transportation, 2006).  Whether that means that effect will interact with the effect of the DSFS remains unknown.

### Classes of Vehicles

#### Description

Classes of vehicles: passenger, truck, multiple, unspecified.  There are four values which the feature, classes of vehicles, can assume: passenger, trucks, multiple, or unspecified. In this context, multiple would mean that at some sites there are just trucks, at other sites just passenger vehicles, and so on.  If only passengers and trucks were measured at all sites, multiple would not be checked. There are clear standards for the definition of these different vehicles and within each class there are many subclasses.  For example, the Federal Motor Carrier Safety Administration has many different subclasses of trucks (e.g., Class A, Class B and Class C) (Federal Motor Carrier Administration, 2014). We relied on the authors to indicate whether passenger vehicles, trucks, or both were being considered in an evaluation.

#### Motivation

Our initial analyses suggest that DSFSs have a larger effect on trucks’ reductions in speed than they do in passenger vehicles’ reductions in speed.  Thus, there is a good reason to include this in the meta-analyses.

### Posted Speeds

#### Description

Posted speed: upstream speed (mph), DSFS speed (mph), downstream speed (mph), multiple, unspecified. There is often only one posted speed limit upstream of the DSFS and at the DSFS.  Sometimes there is one posted speed limit downstream of the DSFS.  However, sometimes there will be several posted speeds upstream or downstream of a DSFS.  We will choose as posted speed limits ones that are closest to the upstream sensor, the DSFS, and the downstream sensor (see below).  If there is not a sensor at one or more of these locations, we will do the following.  If there is not an upstream sensor, we will choose for the upstream posted speed limit that posted speed limit that is closest to a location on the road where the DSFS first becomes visible, but is not yet visible.  Since it is generally the case that a sign with numbers 1 inch in height are visible at 30 feet (Bertucci, 2006), the numerals on a speed limit sign which are 11.7 inches in height (Federal Highway Administration, 2012) should be legible at 386 feet.  If there is not a downstream sensor, we will choose for the downstream posted speed limit the posted speed limit that is closest to the point at which the speed needs to be reduced the most (e.g., for school zone, this would typically be the crosswalk).  By multiple, we mean a study where there are several sites and the posted speed limits at the various locations with respect to the DSFS vary.

#### Motivation

The difference in the posted speed limits upstream of the DSFS and at the DSFS would appear to have a large effect on the change in speed that a DSFS might generate.  This is especially clear in transition zones from a rural highway to a rural local road running through a small town.

### Sensor Types

#### Description

Sensor type: pneumatic, radar, LIDAR, ANPR, multiple, not specified.  There are typically four different ways in which speed is measured: pneumatic tubes, radar, LIDAR, and automatic number plate recognition cameras.

#### Motivation

Municipalities may be interested in whether a particular unit has proven effective in measuring speed reductions in the setting of a DSFS.  It is important to note that the reports of the studies typically do not address the reliability or accuracy of the units that are being used to record speed.

### Mobile Units

#### Description

Mobile units: yes, no (fixed), multiple, not specified.  The DSFS units are almost always mobile, though occasionally they are fixed.

#### Motivation

With locations which require a continuing DSFS presence (in essence a permanent placement), there may be economic and other considerations about whether to use a mobile DSFS or a fixed DSFS.

### Type of Treatments

#### Description

Types of treatment: DSFS, DSFS + police, DSFS + other.  A few studies investigate other interventions in addition to DSFSs, either separately or in conjunction with the DSFS. Since our focus is on DSFSs, we look only at other interventions which piggyback on the DSFS, e.g., a DSFS plus a police car presence.

#### Motivation

If more than one intervention is applied at the same site at the same time or in close succession, this can affect the results and either needs to be captured, or treated as a disqualifying factor for the study.  One may want to know whether, for example, a DSFS together with a police car presence, can significantly reduce speeds beyond what can be obtained with just a DSFS.

## Study Design Characteristics

The studies vary greatly in the way they are designed.  The details of the design are critical to evaluate, not only for those who might wonder why a particular study did or did not show significant results, but also for agencies which may wish to implement their own studies. The characteristics of the studies relevant to the experimental design are discussed below.

### Sensor Positions

#### Description

The sensors that measure speed are positioned differently from one study to the next, and that position appears to have a critical impact on the effect of the DSFS.  There are two descriptors related to the position and number of the sensors.  In addition, Volpe provides a graphic of the sensor, posted speed limit, and DSFS positions where possible in the review of the study included in the Annotated Bibliography.

- Sensor position: upstream of DSFS (yes/no), at DSFS (yes/no), downstream of DSFS (yes/no), multiple, unspecified.

- Number of sensors: upstream, at DSFS, downstream

- Graphic of layout location of sensors with respect to DSFS and other road features.

There are often multiple sensors upstream or downstream of the DSFS, just as there are multiple posted speed limits. Rarely are there multiple sensors at the DSFS. We record the number of sensors upstream of the DSFS, at the DSFS, and downstream of the DSFS (ii).  We also report the positions of an upstream sensor, a sensor at the DSFS, and a downstream sensor.  We will use the same procedure here to choose the sensor position that we used to choose the posted speed limit when we are selecting among several.  Specifically, we will choose the upstream sensor that is at a location upstream of the DSFS which is closest to the point at which the speed limit on the DSFS becomes visible, but is still not recognizable using the standard relation between legibility distance and letter size (Bertucci, 2006).  We will choose as the DSFS sensor the sensor closest to the DSFS (usually at the DSFS). We will choose as the downstream sensor the sensor that is closest to the point where the speeds need to be reduced the most.  Again, we will indicate for which experimental design we are reporting positions (i).

#### Motivation

The sensor positions appear to have a critical impact on the effect of the DSFS.  For example, if the upstream sensor is close to the DSFS, so close that the DSFS is visible, then the effect of the DSFS will be reduced some, or so at least it can be argued.  This information will be important to enter into the meta-analyses when computing the effect of a DSFS.  And it will be important to highlight this information for the state DOTs in the reviews of articles without meta-analyses just in case an effect is unexpectedly small.

### Sites Used to Evaluated Each Treatment

#### Description

The number of sites is the number of different locations where a single treatment was applied and data gathered. Note that with some studies having multiple treatments and multiple sites, it will be important as well to specify the number of sites associated with each treatment.

#### Motivation

The information is typically easy to capture, is a useful descriptor of the study, and is potentially useable in the meta-analysis if the statistics are aggregated across sites or if the information is reported on a site-by-site basis only.

### Before DSFS Activation Measures (Several Features)

#### Description

We are including two descriptors (i, ii) as important to the DSFS before activation:

- Over how long a period is speed measured before the activation of the DSFS: O1(interval)?

- How long before activation of the DSFS were speed measures taken: T1(interval)?

So, for example, if the speed was measured for one week at a point three weeks before the DSFS was activated, we would indicate O1(1 week), T1(3 weeks).  Note that we are entering here information that is also part of the experimental design.  Sometimes there can be multiple experimental designs in a single study.  We will indicate here and with the during and after measure, only the intervals associated with the first experimental design for a particular study (or specify which experimental design is being detailed).  It is too cumbersome here, and in the review, to spell out the information for every experimental design (and not necessary).

#### Motivation

When there is a long lag between the before measure and the during and after measure, the speeds could change (over time) at which the vehicles passed the upstream sensor.  If the DSFS activation hypothesis is measured by the difference in the speeds at the upstream sensor before activation and DSFS sensor during activation, then the long difference in time between the speed measurements upstream and the speed measurements at the DSFS could be a potential confound (e.g., weather might change radically and therefore speeds).

### During DSFS activation measures (Several Features)

#### Description

We recorded four features when the DSFS is activated:

- Over how long a period is speed measured: DSFS(interval, measurement 1, measurement 2, ….)?

- Times after activation of DSFS when speed is measured: DSFS(interval, measurement 1, measurement 2, ….)

- Time of day when speed measured:  AM/PM; and

- Time of day when speed measured: peak/off peak.

The first value of the first feature (i) is the period of time (interval) over which the DSFS is activated. The entries for the values of the second feature (ii) (measurement 1, measurement 2) are the points in time during the activation of the DSFS that measurements are recorded. So, for example, the DSFS might be activated for a month and the speeds would be measured at the end of weeks 1, 2, 3 and 4: DSFS(one month, week 1, week 2, week 3, week 4).  We do not record exactly how long the measurements were taken.  So here, the measurements would have been taken for just the last day of each week or for the entire week.  Note that the last two features (iii, iv) apply not only to the measurement of speeds during activation of the DSFS, but to measurements of speeds at all times and all locations.

#### Motivation

The DSFS may become less effective as the period of time over which it is installed increases.  Thus, it makes sense to record the interval (i).  In addition, the times at which the measurements are taken would appear to be critical (ii).  If the measurement of a one-year installation is taken in the 12th month, that will provide entirely different information about the effectiveness of the DSFS over time than will be provided by a measurement which is taken continuously since the activation.  Both are done.  We indicate the difference by using “month 12” in one case and “continuously” in the second.

### After DSFS Deactivation Measures (Several Features)

#### Description

There are two features (i, ii) that we want to record after the DSFS is deactivated:

- How long a time period after the DSFS was deactivated intervened between deactivation of the DSFS and the measurement of vehicle speeds at the DSFS: T2(interval)?

- How long after DSFS removal/deactivation were the measurements of speed taken: O2(interval)

The interpretations here of these features and the values which each can assume are the same as those described in 2.10.5.1

#### Motivation

The effects of the DSFS once it is deactivated may dissipate as the interval between its deactivation and the measurement of speeds after the deactivation is increased.

### Experimental Design

#### Description

The experimental design is a complex feature. There is no generally accepted way in the literature for designating the experimental design.  It does not take on specific values.  Rather, it depends on the type of study.  For us, it requires at least five components to specify for a single treatment at a single location the details of the feature.  In particular, with just one treatment in a given location, there are three measurement periods: before the DSFS has been activated (before), during the time when the DSFS is activated (during), and after the DSFS is deactivated or removed (after).  And there is the interval of time between the first measurement and the activation of the DSFS and then the interval of time between the deactivation of the DSFS and the second measurement.  These have been discussed above and do not need reiterating here.

In summary, the experimental design when there is one treatment in a given location will include, whenever the information is provided:

- the duration of the period O1(duration) over which measurements are taken of the dependent variable before the DSFS is activated,

- the interval T1(duration) between the end of the before measurement period and the beginning of the activation of the DSFS,

- the period of time for which the DSFS is activated and when measurements of the dependent variable are taken DSFS(duration, measurement 1, measurement 2,…),

- the interval T2(duration) between the end of the activation of the DSFS and the first measurement of the dependent variable after the deactivation, and

- the duration of the period O2(duration) during which measurements are taken after the deactivation.

So, for example, for Cruzado and Donnell (2009), the experimental design is indicated as:

- O1(ns) – T1(ns) -- DSFS(1 week, 1st week) – T2(0) – O2(1 week); and

- O1(ns) – T1(ns) – DSFS(2 weeks, 1st week, 2nd week) – T2(0) – O2(1 week, 2 weeks).

There are two treatments in separate locations.  The above design indicates that neither the duration of the first measurement period nor the time between the first measurement period and the activation of the DSFS were specified (ns): O1(ns), T1(ns).  In the first condition, the DSFS was activated for a week and the measurements of speed were taken during this first week DSFS(1 week, 1st week), and then the measurements after deactivation were taken immediately after, T2(0), for a period of 1 week, O2(1 week).  In the second condition, the DSFS was activated for 2 weeks and measurements of speed were taken during the first week and second week DSFS(2 weeks, 1st week, 2nd week).  The measurements were taken after the DSFS was deactivated exactly, T2(0), one week and two weeks later, O2(1 week, 2 weeks).

One other study will be considered (Walter and Broughton, 2011). There were three conditions in separate locations here:

- O1(1 week) – T1(0) – DSFS(1 week, 1st week) –T2(0) – O2(1 week, 2 weeks);

- O1(1 week) – T1(0) – DSFS(2 weeks, continuously) –T2(0) – O2(1 week, 2 weeks); and

- O1(1 week) – T1(0) – DSFS(3 weeks, continuously) –T2(0) – O2(1 week, 2 weeks).

The difference here was that at some sites the DSFS was operational for one week, for some two weeks, and for some three weeks.  Measurements were taken one week before the DSFS was operational and one and two weeks after the DSFS was deactivated.  Measurements were taken continuously while the DSFS was in operation.  Thus, as compared with the first design, separate measurements were not reported for the first, second and third weeks of operation of the DSFS.

#### Motivation

This feature is relevant for all studies.  It gives a concise overview of the experiment that was conducted. We include durations whenever possible, here, because in the studies reviewed so far these time spans between measurements and over which measurements are taken can vary greatly (from minutes to years). These durations are a good prospect for independent variables in the meta-analysis, as they are frequently available, and it would be very good to know if the independent variables had a significant impact on the effectiveness of the DSFS.

## Vote-Count Summary by Safety Focus

Above, we have presented a vote-count overall (Section 2.6), which provides a summary of the effect of dynamic speed feedback signs on the mean speed, 85th percentile speed and percentage of drivers over the speed limit at the DSFS when it is activated (

Figure 6), downstream of the DSFS when the DSFS is activated (Figure 7), and at the DSFS when it is deactivated. While most of the studies were consistent with the DSFS having a positive effect on safety, some studies indicated there was no effect or, in some cases, a negative effect. It may be that DSFSs work best for one set of safety foci, but not others. Additionally, by identifying the reason that in some cases the DSFS does not produce the desired results, common explanations of why a given implementation did not work other than the safety focus may arise.

Within each different safety focus, we begin with a discussion of the summary of the vote-count with respect to the effect of the DSFS on a reduction in the mean speed, the 85th percentile speed at the DSFS, and the percentage of drivers over the speed limit for hypotheses H1, H2 and H3.  We then provide an example of the installation that is used for each category being discussed (e.g., an example work zone for the safety focus). Details on individual installations are described in the Annotated Bibliography. Finally, within a safety focus, there may be studies which are not consistent with the primary hypothesis, H1. In these cases, we investigate why these studies showed no effect of the DSFS on the mean speed at the DSFS. We chose the mean speed at the DSFS as a focus for our understanding of discrepant results because many more studies analyzed the change in the mean speed at the DSFS after it was activated (H1) than analyzed either the change in the mean speed downstream of the DSFS (H2) or the change in speed at the DSFS after it was deactivated (H3).

Overall, the safety focus most commonly studied was work zones. For all of the speed measures examined and all hypotheses tested, DSFSs reduced driver speed significantly in the overwhelming majority of cases. School zones were next most common, followed by transition zones, curved sections, and straight sections of a roadway. A summary of all the results is presented in Table 6 and analysis of each of the safety foci follows.

Table 6. Count of significant results for reductions in driver speed for the three hypotheses (H1: Activation Effect, H2: Downstream Activation Effect, H3: Deactivation Effect) by speed measure, across the five safety foci.

| Safety Focus | Hypothesis | Measures | Decrease | No Decrease | Proportion |
| --- | --- | --- | --- | --- | --- |
| Work Zone | H1 | Mean Speed | 45 | 7 | 0.87 |
|  |  | 85th Percentile | 30 | 5 | 0.86 |
|  |  | % Over Speed Limit | 37 | 4 | 0.90 |
|  | H2 | Mean Speed | 45 | 19 | 0.70 |
|  |  | 85th Percentile | 29 | 2 | 0.94 |
|  |  | % Over Speed Limit | 32 | 5 | 0.86 |
|  | H3 | Mean Speed | 3 | 0 | 1.00 |
|  |  | 85th Percentile | 3 | 0 | 1.00 |
|  |  | % Over Speed Limit | 3 | 0 | 1.00 |
| School Zone | H1 | Mean Speed | 24 | 4 | 0.86 |
|  |  | 85th Percentile | 17 | 0 | 1.00 |
|  |  | % Over Speed Limit | 16 | 0 | 1.00 |
|  | H2 | Mean Speed | 1 | 3 | 0.25 |
|  |  | 85th Percentile | 1 | 0 | 1.00 |
|  |  | % Over Speed Limit | 0 | 0 | - |
|  | H3 | Mean Speed | 0 | 0 | - |
|  |  | 85th Percentile | 0 | 0 | - |
|  |  | % Over Speed Limit | 0 | 0 | - |
| Transition Zone | H1 | Mean Speed | 29 | 0 | 1.00 |
|  |  | 85th Percentile | 11 | 0 | 1.00 |
|  |  | % Over Speed Limit | 3 | 0 | 1.00 |
|  | H2 | Mean Speed | 1 | 1 | 0.50 |
|  |  | 85th Percentile | 0 | 1 | 0.00 |
|  |  | % Over Speed Limit | 1 | 0 | 1.00 |
|  | H3 | Mean Speed | 0 | 0 | - |
|  |  | 85th Percentile | 0 | 0 | - |
|  |  | % Over Speed Limit | 0 | 0 | - |
| Curved Section | H1 | Mean Speed | 29 | 0 | 1.00 |
|  |  | 85th Percentile | 27 | 0 | 1.00 |
|  |  | % Over Speed Limit | 3 | 0 | 1.00 |
|  | H2 | Mean Speed | 10 | 5 | 0.67 |
|  |  | 85th Percentile | 3 | 0 | 1.00 |
|  |  | % Over Speed Limit | 3 | 0 | 1.00 |
|  | H3 | Mean Speed | 0 | 0 | - |
|  |  | 85th Percentile | 0 | 0 | - |
|  |  | % Over Speed Limit | 0 | 0 | - |
| Straight Zone | H1 | Mean Speed | 3 | 1 | 0.75 |
|  |  | 85th Percentile | 31 | 0 | 1.00 |
|  |  | % Over Speed Limit | 0 | 0 | - |
|  | H2 | Mean Speed | 0 | 0 | - |
|  |  | 85th Percentile | 0 | 0 | - |
|  |  | % Over Speed Limit | 0 | 0 | - |
|  | H3 | Mean Speed | 0 | 0 | - |
|  |  | 85th Percentile | 0 | 0 | - |
|  |  | % Over Speed Limit | 0 | 0 | - |

### Work Zone

The following articles focused in part or in whole on work zones:

- (Bowie, 2003)

- (Brewer, Pestie, & Schneider, 2006)

- (Fontaine & Carlson, 2001)

- (Fontaine, Innovative traffic control devices for improving safety at rural short-term maintenance work zones, 2008)

- (Gambatese & Jafarnejad, 2015)

- (Gambatese & Zhang, 2014)

- (Hajbabie, Medina, Wang, Benekohal, & Chitturi, 2011)

- (Mattox, Sarasua, Ogle, Eckenrode, & Dunning, 2007)

- (McCoy & Pesti, 2001)

- (McCoy, Bonneson, & Kollbaum, 1995)

- (Medina, Benekohal, Hajbabaie, Wang, & Chitturi, 2009)

- (Meyer, 2000)

- (Pesti & McCoy, 2011)

- (Reddy, Datta, Savolainen, & Pinapaka, 2008)

- (Roberts & Smaglik, 2012)

- (Sarasua, Ogle, & Chowdhury, 2006)

- (Teng, Xu, Li, Kwigizile, & Gibby, 2009)

The particular hypotheses and significant findings for each study are reported in Appendix A in Volume II. The information beside each study needs briefly to defined.  As an example, consider the study in the first row of in Table A1 of Appendix A (Bowie, 2003).  There were two analyses undertaken (column labeled “Cases”).  These two analyses were reported for both the mean speed at the DSFS (H1: Mean) and the mean speed downstream of the DSFS (H2: Mean). In both cases, there was a decrease (“-“) in the mean speed.

#### Vote-count

A total of 17 articles were reviewed which focused on the effect of the DSFS in work zones.  With respect to the changes in the mean speed at the DSFS, 53 analyses were undertaken of changes in the mean speed at the DSFS (H1).  Of the 52 analyses, 45 were consistent with a reduction in the mean speed at the DSFS (86.5%). Seven of the analyses indicated no change in the mean speed. These seven analyses were spread over three studies (Gambatese & Zhang, 2014; Fontaine, 2008; Sarasua, Ogle, & Chowdhury, 2006) (Figure 12). The relative number of studies favoring H2 (85.7%) and H3 (90.2%) were roughly consistent with those favoring H1.

Note that the information in Appendix A can be used to determine which studies contributed to which count. Consider the studies that showed no change in the mean speed at the DSFS (H1).  Looking at “H1: Mean” and “0” we see that three of the analyses were reported in one study (Fontaine, 2008), two in a second study (Gambatese & Zhang, 2014), and the final two in a third study (Sarasua, Ogle, & Chowdhury, 2006).

Figure 12.  Work Zones: H1

With respect to changes in the mean speed at the downstream sensor, while the DSFS was activated, a total of 64 analyses were reported (H2). Of these, 45 were consistent with a decrease in the mean speed at the downstream sensor (70.3%), 13 with no change in the mean speed (20.3%), and 6 with an actual increase in the mean speed (9.4%). Similar changes are noted in the 85th percentile speed and the percentage of drivers traveling over the speed limit (Figure 13).

Figure 13.  Work Zones: H2

Finally, with respect to changes in the mean speed at the DSFS after the DSFS is deactivated, there were very few such analyses, in fact only three such analyses (H3).  All showed a halo effect (a continued reduction in the mean speed of the DSFS after deactivation below what it was before activation).  The halo effect existed as well for the 85th percentile speed and the percentage of drivers traveling over the speed limit (Figure 14).

Figure 14.  Work Zones: H3

#### Implementation Characteristics

An example of an implementation of a mobile DSFS in a work zone is displayed in Figure 15 (Gambatese & Jafarnejad, 2015).  The mobile unit was mounted on a truck.  A number of different maintenance operations were being undertaken at the time the mobile unit was present including: (a) inspecting light poles with a stop at each light pole and a lane closure and (b) cleaning the drains along the right shoulder of the roadway, also with stops but without a complete lane closure. Within the maintenance work zone, the truck equipped with the DSFS moved at a slow speed (usually 5-10 mph) behind the maintenance equipment conducting the work to provide a warning to approaching vehicles about the work zone and to advise drivers of a decrease in speed (Shadow Vehicle 2 in Figure 15).  Importantly, there was a vehicle upstream of the vehicle with the DSFS which provided warning information (either a four-dot pattern or an arrow when encroaching a lane with a narrow shoulder; Shadow Vehicle 1 in Figure 15.).  Additionally, the work vehicle itself had the capability of displaying the same information as the Shadow Vehicle 1. These additional vehicles are unlikely to influence the effect of the activation of the DSFS at the DSFS (H1) or the deactivation of the DSFS at the DSFS (H3).  But, there is reason to believe that the presence of the information on Shadow Vehicle 1 and the work vehicle may have influenced the speed of drivers downstream of the DSFS when the DSFS was activated.

Figure 15.  Example work zone implementation of DSFS. (Gambatese & Jafarnejad, 2015)

#### Exceptions

There were three studies which reported no change in speeds in some of the sites at the DSFS (H1).  First, in one study there was no statistically significant effect of the DSFS on the mean speed at each of the two sites analyzed in the study (Gambatese & Zhang, 2014).  In this study, the DSFS was evaluated by itself only on Day 3 of the first case study (on other days, the DSFS was evaluated in combination with a portable changeable message sign, PCMS, which displayed messages such as, “slow for workers,” “workers in roadway,” and “narrow lane”).  There were two travel lanes in one direction on a section of highway which was being paved.  The two lanes tapered to one lane where the paving began somewhere downstream.  On Day 3 of the first case study, the contractor paved the roadway shoulder and fully closed the slow lane, moving the passing traffic farther away from the actual work taking place. One might expect that the vehicle speeds would be greater in this case than if the work were directly adjacent to the travel lane. That is, when a closed, “buffer” lane is provided, vehicle speeds tend to increase.

In a second study, the results were mixed (Fontaine, 2008).  There were two sites.  The speeds of trucks and cars were measured separately.  Thus, there are four measurements of speed over the two sites.  Broken out by vehicle class, significant reductions in speed were observed at only one of two sites for trucks.  No significant reductions in speed were observed for cars.  However, importantly, at all five sensors, the speeds were lower for all vehicles when the DSFS was present.  At the location with a reverse curve, the reduction in speed for cars and trucks at the curve when the DSFS was present compared to when standard work zone signage was present was large, respectively -9 and -7 mph.  At the location with an activity area, the reduction in speed was smaller for cars than it was for trucks, but still in evidence (respectively -1.4 and -9.8 mph).  The authors themselves suggest that it may be the location of the DSFS which is critical.  Also, a factor here is that the observations were collected for only one day, thus making a small effect size difficult to detect.

In the third study, only two of the 17 sites that were analyzed showed no reduction in speed (Sarasua, Ogle, & Chowdhury, 2006). These occurred in maintenance sites where the driver’s speed was displayed along with a changeable message sign which told the driver to stay alert if the driver was traveling at or below the speed limit and displayed one of two signs depending on whether a police officer was posted downstream of the sign: “Watch Speed” – no officer – or “Fines Ahead” – officer present. The authors themselves present a convincing case for why speeds were not reduced significantly in the maintenance sites: “A primary reason for this is the inability of the vehicles to speed in the maintenance sites studied due to lane closures, shifting of traffic, and the maintenance activity. The speed reduction range… was from 0.20 to 3.7 mph. In many cases, the actual speeds of vehicles traveling through the maintenance work zone were not high enough to trigger the warning message and thus the vehicles tend to travel at their original speed.”

In summary, the seven exceptions to the finding of a decrease in the mean speed at the DSFS when it was activated are all arguably reasonable ones. A buffer lane may well serve to decrease the effect of a DSFS (Gambatese & Zhang, 2014).  Small sample sizes could easily reduce the power of the statistical test (Fontaine, 2008).  And when there is very little ability to speed in a maintenance site as a function of the road geometry, a DSFS would not be expected to slow down the vehicles by much if anything (Sarasua, Ogle, & Chowdhury, 2006).   Moreover, it is worth noting that in all seven exceptions there was a non-statistically significant decrease in speed.

### School Zone

The following articles focused in part or in whole on school zones:

- (Jeihani, Ardeshiri, & Naeeni, 2012)

- (Lee, Lee, Choi, & Oh, 2005)

- (O'Brien & Simpson, 2012)

- (Saito & Ash, 2005)

- (Ullman & Rose, 2005)

The particular hypotheses and significant findings for each of the studies on school zones are in Table A2 of Appendix A in Volume II.

#### Vote-count

A total of five studies were reviewed which focused on school zones.  With respect to the mean speed at the DSFS, a significant decrease in speed was recorded at 12 sites and an increase in speed at three sites (Saito & Ash, 2005) (H1).  Importantly the measures that captured the drivers who were traveling the fastest, the 85th percentile measures and the percentage of drivers of drivers over the speed limit measures, showed significant decreases in speed as well except for one site which showed no change in the 85th percentile speed (Ullman & Rose, 2005).

Figure 16. School Zone: H1

Two studies look at the change in speed downstream of the DSFS after it was activated (H2).  One showed a decrease at the one site analyzed (Lee, Lee, Choi, & Oh, 2005).  The other showed increases at the three sites analyzed (Jeihani, Ardeshiri, & Naeeni, 2012).  One study reported a decrease in the 85th percentile speed (Lee, Lee, Choi, & Oh, 2005).

Figure 17.  School Zone: H2

There were no studies which examined the halo effect (H3).

#### Implementation Characteristics

An illustrative DSFS in a school zone is displayed below in Figure 18 (Saito & Ash, 2005).  The signs were activated only during the time the school was in session.  Small LED lights in the numbers would flash if drivers were traveling above the speed limit (25 mph).

Figure 18. DSFS in school zone  (Saito & Ash, 2005).

#### Exceptions

In the one study that showed an increase in speed at three of the eight locations, clear reasons for the possible increase were given for two of the locations (Saito & Ash, 2005). At one of the locations, drivers were already traveling on average below the speed limit.  The introduction of the DSFS increased the speed significantly, but only very slightly. Still, this is an unintended consequence which should be considered before installing a DSFS.  At the second of the two locations, the DSFS operated only intermittently because it was solar powered.  There is no obvious explanation for why there was an increase in the speed at the third of the three sites.  So, we looked at the characteristics of the DSFS.  Small dots flashed if the driver was traveling 5 mph over the speed limit.  However, the speed itself did not flash. Such dots may not be effective as an indication that the driver is above the speed limit. Across the four measurement periods during the day, the increase was 1.2 mph.

### Transition Zone

The following articles focused in part or in whole on transition zones:

- (Cruzado & Donnell, 2009)

- (Hallmark, et al., 2007)

- (Hallmark & Hawkins, 2015)

- (Hallmark, Knickerbocker, & Hawkins, 2013)

- (Kamyab, Andrle, & Kroeger, 2002)

- (Schoenecker, Sandberg, Sebastian, & Soler)

- (Ullman & Rose, 2005)

- (Williamson & Fries, 2015)

The particular hypotheses and significant findings for each of the studies on school zones are in the Appendix C, in Volume II.

#### Vote-Count

Of the 31 separate analyses of the change in the mean speed at the DSFS when it is activated in transition zones, all reported a significant reduction in speed.  Eleven of the 13 analyses of the 85th percentile speed at the DSFS reported a significant reduction.  In two analyses, no change was noted (Ullman & Rose, 2005).  All five analyses of the percentage of travelers over the speed limit reported a significant reduction.

Figure 19.  Transition Zone: H1

There were too few studies of the effect of the DSFS on the reduction in speed downstream of the DSFS to draw general conclusions regarding transition zones. With respect to the mean speed, the studies were split, one indicating a reduction (Williamson & Fries, 2015), one no change (Kamyab, Andrle, & Kroeger, 2002).  With respect to the 85th percentile speed, the one analysis available suggested no change in the speed downstream of the DSFS. (Kamyab, Andrle, & Kroeger, 2002) Finally, with respect to the percentage of travelers over the speed limit, the one analysis reported a reduction (Kamyab, Andrle, & Kroeger, 2002).

Figure 20.  Transition Zone: H2

There were no analyses of the halo effect (H3).

#### Implementation Characteristics

An example implementation of DSFS in a transition zone is given below in Figure 21.  In this study, dynamic feedback speed signs were installed at 12 sites in rural Pennsylvania (Cruzado & Donnell, 2009).  The sites included both curved and straight sections of roadway where a transition was being made from a two-lane highway to a rural community.  The posted speed limits of the highway upstream of the transition zone were between 45 mph and 55 mph.  The posted speed limits in the transition zone were between 25 mph and 35 mph.  Figure 21 shows the distance of the speed limit signs to the DSFS (labeled DSDS in the figure) and the location of the sensors and their distance from the DSFS. Often this information needed to be determined from a description written purely in text which could be difficult to piece together.

Figure 21.  Data collection site setup.  (SL = speed limit sign).  (Cruzado & Donnell, 2009)

#### Exceptions

There were no exceptions.  All analyses reported of change in the mean speed at the DSFS when the DSFS was activated indicated a significant reduction.  So we went on to examine the two analyses in which no change was observed in the 85th percentile speed at the DSFS  (Ullman & Rose, 2005).  The posted speed limit at the two sites where the DSFS was to be installed was 45 mph.  The average speed at the two sites was considerably higher at one, 55.2 mph, and only slightly higher at the second, 47.7 mph.  The 85th percentile speeds at the two sites reflected the difference in the mean speeds, 61 mph at the first site and 53 mph at the second site.  The reduction in the mean speed was significant at both sites, from 55.2 mph to 51.8 mph and from 47.7 mph to 45.1 mph.  The reduction in the 85th percentile speeds went from 61 mph to 58 mph and from 52 mph to 49 mph at the second site.  Perhaps with more observations the reductions in the 85th percentile speed would have been significant.  Still, it is notable how much higher the mean and 85th percentile speeds are at the first site.  So we went on to look at the characteristics of the DSFS, suspecting that perhaps it was not a flashing unit. In the meta-analysis we analyze the effect of the different characteristics of the DSFS on the various functions of speed.

### Curved Section

The following articles focused in part or in whole on curved sections of roadways:

- (Bertini, Monsere, Nolan, Bosa, & Abou El-Seoud, 2006)

- (Bullough, Skinner, Brons, & Rea, 2012)

- (Drakopoulos & Uprety, 2003)

- (Hallmark, Hawkins, & Smadi, 2015)

- (Knapp & Robinson, 2012)

- (Tribbett, McGowen, & Mounce, 2000)

- (Ullman & Rose, 2005)

- (Western Transportation Institute, 2003)

The particular hypotheses and significant findings for each of the studies on curved sections are in Appendix A of Volume II.

Vote-count. A total of 34 analyses were undertaken of the change in the mean speed at the DSFS after the DSFS was activated on curved sections of roadways.  Of these, there was a significant reduction in the mean speed in 32 analyses and no change in the mean speed in the other two analyses (Ullman & Rose, 2005).  A similar distribution of results was seen in the analyses of the 85th percentile speed.  Twenty-seven analyses showed a decrease in the 85th percentile speed while four showed no change (Ullman & Rose, 2005).  There were many fewer analyses of the change in the percentage of drivers exceeding the speed limit.  Five of the analyses indicated a reduction in this percentage, two indicated no change (Ullman & Rose, 2005).

Figure 22.  Horizontal Curves: H1

Relatively speaking, there was less support for a decrease in speed downstream of the DSFS after it was activated, eleven analyses showing a decrease and five showing no change (Tribbett, McGowen, & Mounce, 2000).  The three analyses of the 85th percentile speed and the percentage of drivers traveling over the speed limit showed a decrease.

Figure 23.  Horizontal Curves:  H2

There were no analyses of the effect of deactivating the DSFS on the speed at the DSFS (H3).

#### Implementation Characteristics

A typical implementation of a DSFS at a curved section is presented below in Figure 24 (Bertini, Monsere, Nolan, Bosa, & Abou El-Seoud, 2006).   The DSFS was a two-phase changeable message sign (CMS).  The default message for vehicles whose speed was less than 50 mph in the first and second phases was, respectively, “Caution” and “Sharp Curves Ahead.”  The warning message for vehicles traveling between 50 mph and 70 mph in the first and second phases was, respectively, “Slow Down” and “Your Speed is XX MPH”.  Finally, the excessive speed message for vehicles traveling over 70 mph in the first and second phases was respectively, “Slow Down” and “Your Speed is Over 70 MPH”.

Figure 24.  Curved section DSFS.  (Before on left; after on right).  (Bertini, Monsere, Nolan, Bosa, & Abou El-Seoud, 2006)

#### Exceptions

The two exceptions to the reduction in mean speed at the DSFS after the DSFS was activated on curved sections were reported by Ullman & Rose (2005).  In their study, they found no change in the mean speed.  Interestingly, they also reported negative results in transition zones.  Although they did find a reduction in the mean speed in transition zones, they found no reduction in the 85th percentile speed.  Here, they are finding no statistically significant reductions in the mean speed.  We note, as before, that the DSFS they used reported only the speed.  It did not change as a function of the driver’s speed with respect to the posted speed limit.  However, there is another, more obvious explanation.  The failure to find a reduction in the mean speed was confined to trucks.  The trucks were already traveling very near the posted speed, 30 mph.  At one location they were traveling exactly at that speed before the DSFS was activated.  In the other location they were traveling 31.5 mph.

### Straight Section

The following articles focused in part or in whole on straight sections of roadway:

- (Bloch & Automobile Club of Southern California, 2007)

- (Chang, Nolan, & Nihan, 2004)

- (City of Englewood, CO, 2014)

The particular hypotheses and significant findings for each of the studies on straight sections are in the Table A5 of Appendix A in Volume II.

#### Vote-count

There were only three studies reporting information on straight (tangent) sections, largely in residential neighborhoods.  One study did not report any statistical analyses (City of Englewood, CO, 2014).  With regard to the effect on speed at the DSFS (H1), one study reported a decrease in both the mean speed and the percentage of drivers over the speed limit at two sites (Bloch & Automobile Club of Southern California, 2007).  No analysis was made of the change in mean speed in the other study (Chang, Nolan, & Nihan, 2004).

Figure 25.  Straight Section: H1

With regard to the effect of the activation of the DSFS on speed downstream of the DSFS (H2), reductions were reported in the mean speed, the 85th percentile speed and the percentage of drivers over the speed limit except for one site.  At that site, there was an increase in speed downstream of the DSFS after it was activated (Chang, Nolan, & Nihan, 2004).

Figure 26.  Straight Section: H2

Finally, one study looked at the change in the speed at the DSFS after it was deactivated, comparing it with the speed before it was activated (Bloch & Automobile Club of Southern California, 2007).  There was no halo effect as indicated by no change in speed between the before and after periods at the DSFS.

Figure 27.  Straight Section: H3

#### Implementation Characteristics

A typical layout for a DSFS on a residential street is displayed below in Figure 28 (Chang, Nolan, & Nihan, 2004).  The DSFS is located directly below the speed limit sign.

Figure 28.  (Chang, Nolan, & Nihan, 2004)

#### Exceptions

No studies indicated a failure to produce a decrease in speed at the DSFS when it was activated (H1).  Given that, we went on to look at possible exceptions to H2.  One study found an increase in the speed downstream of the DSFS after it was activated (Chang, Nolan, & Nihan, 2004). Again, we asked why this might have occurred.  The authors speculate that it is the fact that this location already had lower speeds: “Because of this sign’s close proximity to Helen Keller Elementary School, this location had the lowest average speed of the four locations before installation. It can be speculated that drivers were already respecting the rules of the road, and this suggests that these signs may be better suited for locations with higher average speeds from the outset.”  However, another explanation is possible.  It appears that there is an outlier in the before data in the month of April.  The speeds in the three months prior to the activation of the DSFS were 21.9 (April 2001), 27.6 (February 2002) and 27.0 mph (June 2002).  The speeds in the four months during activation were 26.3 (June 2002), 25.9 (August 2002), 26.2 (January 2003) and 25.4 mph (April 2004).  The authors did not explore the reason for this possible outlier.

## Discussion

### Overall

Overall, there is strong support for the effectiveness of dynamic speed feedback signs when a vote-count is used as the basis for support. Signs reduce the mean speed, the 85th percentile speed and the percentage of drivers traveling above the speed limit at the DSFS (H1,

Figure 6), and downstream of the DSFS in large numbers (H2, Figure 7).  There are indications of a significant halo effect after the DSFS is deactivated, although the sample size is limited to three studies (H3).

Perhaps not surprisingly, the support for the effect of the DSFS on speeds downstream of the DSFS is less strong than support for the effect at the DSFS.  Fully 92% of the analyses show a significant reduction in speed at the DSFS whereas only 68% of the analyses show a significant reduction in speed downstream of the DSFS. Importantly, this support is equally strong both adjacent to and downstream of the DSFS for reductions in the 85th percentile speed and reductions in the percentage of drivers traveling over the speed limit (Table 6).  These are the drivers most likely to be involved in a crash which leads to injuries or fatalities.

### Safety Focus

We identified five primary safety foci: work zones, school zones, transition zones, curved sections and straight sections.  There was a clear variation in the effectiveness of the DSFS on the reduction in speed at the DSFS (H1), with the smallest proportion of studies showing an effect (80%) in school zones and the most in transition zones (100%, Table 6).

The finding that only 80% of studies showed a significant effect in school zones results partly from the sample size of studies, and particular design features of the studies showing no significant effect.  Fifteen studies looked at the effect of the DSFS at school zones, of which three reported negative results, all from the same publication (Saito & Ash, 2005).  However, two of these negative results were easily explained.  In one case, vehicles were already traveling at the posted speed limit. In the other case, the signs were not functioning consistently across days given that they were solar powered.  In the third case, the speed did not flash if drivers were traveling above the posted speed limit. Instead lights flashed above either side of the posted speed limit. Regardless, as was noted above, the change in speed, although not reduced, with the DSFS in place was minimal.

More generally, three factors appeared to reduce the effect of the DSFS on the speed of vehicles.  First, it would appear that a DSFS will be less effective at reducing the speeds of trucks than of automobiles.  However, this is because the trucks are already traveling at the posted speed limits, thus there is little reason to reduce speed (Ullman & Rose, 2005).  Second, it was noticed that a DSFS was less effective if the geometry of the roadway moved the potential for conflict further away from the drivers.  So, for example, in one case, a work zone, the travel lane was separated from the workers by a buffer lane (Gambatese & Zhang, 2014).  Drivers may well have felt that there was a safe margin separating them from the workers and therefore felt less of a need to reduce their speed.  Finally, if drivers, not just trucks, were already traveling at the posted speed limits the DSFS was not likely to reduce the speed of the drivers (Saito & Ash, 2005). In fact it could increase it, though still keeping it within the posted speed limits.

### Limitations

The primary limitation of a review like this is not what it would appear at first glance. At first glance, at least to some, it would appear that the question asked is a simple one: Do DSFSs reduce speed?  However, it turns out that the answer depends on how one interprets that particular question (13 different hypotheses were identified), what dependent variable was used (three dependent variables were included), and then a number of other factors that were sometimes mentioned and sometimes not (where the sensors were located with respect to the DSFS, the exact characteristics of the DSFS).

Ultimately, what is most encouraging is that despite the large variation in so many of the critical details that define each study, the results overall, as measured by a vote-count, clearly support the use of DSFSs.  The Meta-Analysis (Section 3) informs which of these various details are the most likely to impact the various functions of speed for each of the three different ways of evaluating the effect of a dynamic speed feedback sign.

## References

Bates, D., Maechler, M., Bolker, B., & Walker, S. (2015). Fitting Linear Mixed-Effects Models Using lme4. Journal of Statistical Software, 67(1), 1-48. doi:10.18637/jss.v067.i01

Bertini, R., Monsere, C., Nolan, C., Bosa, P., & Abou El-Seoud, T. (2006). Field evaluation of Myrtle Creek advanced curve warning system. Washington, DC: Federal Highway Administration.

Bertucci, A. (2006). Sign Legibility Rules of Thumb. Bristol, PA: United States Sign Council. Retrieved June 13, 2016, from http://www.usscfoundation.org/USSCSignLegiRulesThumb.pdf

Bloch, S., & Automobile Club of Southern California. (2007). Comparative study of speed reduction effects of photo-radar and speed display boards. Transportation Research Record, 1640.

Bowie, J. (2003). Efficacy of speed monitoring displays in increasing speed limit compliance in highway work zones (Masters's Thesis). Salt Lake City: Brigham Young University.

Brewer, M., Pestie, G., & Schneider, W. (2006). Improving compliance in work zone speed limits. Transportation Research Record, 67-76.

Bullough, J., Skinner, N., Brons, J., & Rea, M. (2012). Using Lighting And Visual Information to Alter Driver Behavior. Albany, NY: New York State Department of Transportation.

Chang, K., Nolan, M., & Nihan, N. (2004). Radar speed signs on neighborhood streets: An effective traffic calming device? 2004 ITE Annual Meetingn. Lake Buena Vista, FL.

Christ, S., & Abrams, R. (2006). Abrupt onsets cannot be ignored. Psychonomic Bulletin & Review, 13, 875-880.

City of Bellevue Transportation Department. (2009). Stationary radar sign program 2009 report. Bellevue, WA: Author.

City of Englewood, CO. (2014). Recent accomplishments: Traffic safety improvements at Logan and Eastman. Englewood, CO: Author. Retrieved January 3, 2017, from http://www.englewoodgov.org/inside-city-hall/boards-and-commissions/transportation-advisory-committee/recent-accomplishments

Cooper, H., Hedges, L., & Valentine, J. (2009). The Handbook of Research, Synthesis and Meta-analysis (2nd ed.). New York: Russell Safe.

Cruzado, I., & Donnell, E. (2009). Evaluating effectiveness of dynamic speed display signs in transition ones of two-lane, rural highways in Pennsylvania. Transportation Research Record, 1-8.

Donnell, E., & Cruzado, I. (2007). Effectiveness of speed minder in reducing driving speeds on rural highway in Pennsylvania: Literature synthesis. State College, PA: Pennsylvania Transportation Institute.

Drakopoulos, A., & Uprety, S. (2003). I-43 speed warning sign evaluation. Milwaukee, Wisconsin: Marquette University.

Federal Highway Administration. (2012, October 19). Flexibility in Highway Design. Chapter 3. Functional Classification. Retrieved June 8, 2016, from http://www.fhwa.dot.gov/environment/publications/flexibility/ch03.cfm

Federal Highway Administration. (2012). Manual on Uniform Traffic Control Devices: Standard Hihway Signs (2012 Supplement). Washington, DC: Federal Highway Administration. Retrieved June 13, 2016, from http://mutcd.fhwa.dot.gov/shsm_interim/#rs

Federal Highway Administration. (2013). Highway functional classification concepts, criteria and procedures. Washington, DC: Federal Highway Administration.

Federal Motor Carrier Administration. (2014, March 27). Commercial drivers licences: Drivers. (Federal Motor Carrier Safety Administration) Retrieved June 8, 2016, from https://www.fmcsa.dot.gov/registration/commercial-drivers-license/drivers

Fontaine, M. (2008). Innovative traffic control devices for improving safety at rural short-term maintenance work zones. College Station, TX: Texas Transportation Institute.

Fontaine, M., & Carlson, P. (2001). Evaluation of speed displays and rumble strips in rural-maintenance work zones. Transportation Research Record, 1745, 27-38.

Gambatese, J., & Jafarnejad, A. (2015). Evaluation of Radar Speed Display for Mobile Maintenance. Salem, Oregon: Oregon Department of Transportation Research Unit.

Gambatese, J., & Zhang, F. (2014). Safe and effective speed reductions for freeway work zones Phase 2. Salem, OR: Oregon Department of Transportation.

Glass, G. (1976). Primary, secondary, and meta-analysis of research. Educational Researcher, 5(10), pp. 3-8.

Hajbabie, A., Medina, J., Wang, M., Benekohal, R. F., & Chitturi, M. (2011). Sustained and halo effects of various speed reduction treatments in highway work zones. Transportation Research Record, 118-128.

Hallmark, S., & Hawkins, H. K. (2015). Use of DSFS as a Speed Transition Zone Countermeasures in Small, Rural Communities. 2015 IEEE 18th International Conference on Intelligent Transportation Systems (pp. 1448-1454). Canary Islands, Spain: IEEE Computer Society.

Hallmark, S., Hawkins, N., & Smadi, O. (2015). Evaluation of dynamic speed feedback signs on curves: A national demonstration project. McClean, VA: Office of Infrastructure Research and Development, Federal Highway Administration.

Hallmark, S., Knickerbocker, S., & Hawkins, N. (2013). Evaluation of Low Cost Traffic Calming for Rural Communities -- Phase II [Updated]. Ames: Institute for Transportation, Iowa State University.

Hallmark, S., Peterson, E., Fitzsimmons, E., Hawkins, N., Resler, J., & Welch, T. (2007). Evaluationn of gateway and low-cost traffic-calming treatments for major routes in small, rural communities. Ames: Center for Transportation Research and Education, Iowa State University.

Hedges, L., & Vevea, J. (1998). Fixed- and random-effects models in meta-analysis. Psychological Methods, 3, 486-504.

Jeihani, M., Ardeshiri, A., & Naeeni, A. (2012). Evaluating the effectiveness of dynamic speed display signs. Baltimore, MD: Morgan State University National Transportation Center.

Kamyab, A., Andrle, S., & Kroeger, D. (2002). Methods to reduce traffic speed in high pedestrian areas. St. Paul: Minnesota Department of Transportation.

Kloeden, C., McLean, A., Moore, V., & Ponte, G. (1997). Traveling speed and the risk of crash involvement. Adelaide, Australia: NHMRC Road Accident Research Unit, The University of Adelaide.

Knapp, K., & Robinson, F. (2012). The vehicle speed impacts of a dynamic horizontal curve warning sign on low-volume local roadways. St. Paul: Minnesota Department of Transportation.

Koricheva, J., & Gurevitch, J. (2016). Place of Meta-analysis among Other Methods of Research Synthesis. In J. Koricheva, J. Gurevitch, & K. Mengersen (Eds.), Handbook of Meta-analysis in Ecology and Evolution (pp. 3-13). Princeton, NJ: Princeton University Press.

Lee, C., Lee, S., Choi, B., & Oh, Y. (2005). Effectiveness of speed-monitoring displays in speed reduction in school zones. Transportation Research Record, 27-35.

Mattox, J., Sarasua, W., Ogle, J., Eckenrode, R., & Dunning, A. (2007). Development and evaluation of speed-activated sign to reduce speeds in work zones. Transportation Research Record, 3-11.

McCoy, P., & Pesti, G. (2001). Smart work zone technology evaluations: speed monitoring displays and condition-responsive, real-time travel information systems. Lincoln: Mid-America Transportation Center, University of Nebraska-Lincoln.

McCoy, P., Bonneson, J., & Kollbaum, J. (1995). Speed reduction effects of speed monitoring displays with radar in work zones on interstate highways. Transportation Research Record, 65-72.

Medina, J., Benekohal, R., Hajbabaie, A., Wang, M., & Chitturi, M. (2009). Downstream effects of speed photo-radar enforcement and other speed reduction treatments on work zones. Transportation Research Record, 24-33.

Meyer, E. (2000). Evaluation of two strategies for improving safety in highway work zones. Mid-Continent Transportation Symposium 2000 Proceedings (pp. 62-66). Ames: Iowa State University.

National Highway Traffic Safety Administration. (2014). Traffic safety facts 2012 data: Speeding. Washington, D.C.: Author.

National Highway Traffic Safety Administration. (2017, May). Traffic Safety Facts. 2015 Data. Speeding. Retrieved June 13, 2016, from https://crashstats.nhtsa.dot.gov/Api/Public/ViewPublication/812409

NHTSA. (2010). The Economic and Societal Impact of Speeding. Washington, D.C.: National Highway Transportation Safety Administration.

NHTSA. (2017). Traffic Safety Facts: 2015. Washington, D.C.: National Highway Transportation Safety Administration.

Nilsson, G. (2004). Traffic Safety Dimension and the Power Model to describe the Effect of Speed on Safety. Sweden: Lund Institute of Technology.

O'Brien, S., & Simpson, C. (2012). Use of "Your Speed" changeable message signs in school zones: Experience from North Carolina Safe Routes to School Program. Transportation Research Record.

Pesti, G., & McCoy, P. (2011). Long-term effectiveness of speed monitoring displays in work zones on rural interstate highways. Transportation Research Record, 21-30.

R Core Team. (2016). R: A language and environment for statistical computing. Vienna, Austria. Retrieved from https://www.R-project.org/

Raudenbush, S. (2009). Analyzing effect sizes: Random effects models. In H. Cooper, L. Hedges, & J. Valentine, The Handbook of Research Synthesis and Meta-analysis (pp. 295-315). New York: Russell Sage Foundation.

Reddy, V., Datta, T., Savolainen, P., & Pinapaka, S. (2008). Evaluation of innovative safety treatments: A study of the effectiveness of motorist awareness systme in construction work zones. Tallahassee: Florida Department of Transportation.

Richard, C., Campbell, J., Lichty, M., Brown, J., Chrysler, S., Lee, J., . . . & Reagle, G. (2013). Motivations for speeding, Volume II: Findings report. Washington, DC: National Highway Traffic Safety Administration.

Roberts, C., & Smaglik, E. (2012). Driver feedback on monetary penalty and its impact on work zone speed. Transportation Research Record, 27-34.

Robinson, F. (2012). The vehicle speed impacts of dynamic horizontal curve warning sign on low-volume local roadways. St. Paul: Minnesota Department of Transportation.

Saito, M., & Ash, K. (2005). Evaluation of four recent traffic safety initiatives, Volume IV: Increasing speed limit compliance in reduced speed school ones. Salt Lake City, Utah: Utah Department of Transportation Research Division.

Santiago-Chaparro, K., Chitturi, M., Bill, A., & Noyce, D. (2012). Spatial effectiveness of speed feedback signs. Transportation Research Record, 8-15.

Sarasua, W., Ogle, J., & Chowdhury, M. (2006). Better management of speed control in work zones. Columbia, SC: South Carolina Department of Transportation.

Schoenecker, T., Sandberg, W., Sebastian, K., & Soler, P. (n.d.). Long-term effectiveness of dynamic speed monitoring display (DSMD) signs for speed management at speed limit transitions.

Schroeder, P., Kostynuik, L., & Mack, M. (2013). 2011 National survey of speeding attitudes and behaviors. Washington, DC: National Highway Traffic Safety Administration.

South Carolina Department of Transportation. (2006). Traffic Calming Guidelines. Columbia, SC: Soutch Carolina Department of Transportation. Retrieved June 13, 2016, from http://safety.fhwa.dot.gov/speedmgt/ref_mats/fhwasa09028/resources/SCDOT%20Traffic%20calming%20guidelines.pdf

Tefft, B. (2011). Impact speed and a pedestrain's risk of severe injury or death. Washington DC: AAA FTS.

Teng, H., Xu, X., Li, X., Kwigizile, V., & Gibby, A. (2009). Evaluation of speed monitoring displays for work zones in Las Vegas, Nevada. Transportation Research Record, 45-56.

Transportation Research Board. (2010). Highway Capacity Manual (2010). Washington,DC: National Academies of Science.

Tribbett, L., McGowen, P., & Mounce, J. (2000). An evaluation of dynamic curve warning systems in the Sacramento River Canyon. Bozeman, MT: Western Transportation Institute. Retrieved December 13, 2016, from http://www.coe.montana.edu/ce/patm/pubs/files/2000curve.pdf

Tribbett, L., McGowen, P., & Mounce, J. (2000). An evaluation of dynamic curve warning systems in the Sacremento River Canyon. Bozeman, MT: Western Transportation Institute.

Turner, J. (2015, June 10). What's a Work Zone? Retrieved June 8, 2016, from Federal Highway Administration Research and Technology: Public Roads: http://www.fhwa.dot.gov/publications/publicroads/99mayjun/workzone.cfm

Ullman, G., & Rose, E. (2005). Evaluation of dynamic speed display signs. Transportation Research Record, 92-97.

van Houwelingen, H., Arends, L., & Stijnen, T. (2002). Advanced methods in meta-analysis: Multivariate approach and meta-regression. Statistics in Medicine, 21(4), pp. 589-624.

Viechtbauer, W. (2010). Conducting meta-analyses in R with the metafor package. Journal of Statistical Software, 36(3), 1-48. Retrieved from http://www.jstatsoft.org/v36/i03/

Western Transportation Institute. (2003). Greater Yellowstone Rural ITS Project: Work Order II-2C Dynamic Warning VMS Evaluation of Wyoming Site. Bozeman, MT: Western Transportation Institute.

Williamson, M., & Fries, R. (2015). Effectiveness of radar speed signs in a university environment. ite Journal, 44-48.

# Meta-Analysis

## Summary

Each study which measured the effectiveness of DSFS was assessed for availability of the underlying data on vehicle speeds. These data were compiled, along with accompanying data on the location and timing of the speed measurements, vehicle type, safety focus, and any other relevant data. The compiled database was then used to carry out a statistical analysis across all the published studies, known as a meta-analysis, to evaluate the size of the speed reduction effects.

For each of the three major hypotheses, the Activation Hypothesis, Downstream Hypothesis, and Deactivation Hypothesis, statistically significant reductions in speed were detected. The size of the effect ranged from 2-4 mph, with the strongest effects shown for the activation effect for passenger cars (4.14 mph reduction). Safety foci, surprisingly, did not show consistent differences in the size of the speed reduction. For the activation effect, the smallest speed reduction within a safety focus was for horizontal curves (2.27 mph) and the largest for school zones (3.21 mph, Table 10).

Overall, these results indicate that DSFS installations were consistently effective in reducing driver speeds. Speeds generally increased downstream of DSFS installations, again indicating that the location of the installation sees a clear effect on driver behavior. Following removal of the DSFS, the few studies available indicated that vehicle speeds at the former site of the DSFS remained lower than prior to the installation, giving evidence for a ‘halo effect’ following removal of temporary DSFS installations.

## Introduction

Following the Literature Review, it is clear that sufficient publications were available in the literature to carry out a statistical analysis of DSFS effectiveness. To do so, publications were reviewed for availability of the underlying data on vehicle speeds, in tabular or graphical form, and data were compiled in a database. The data were then analyzed using meta-analysis approach. Meta-analysis is a statistical technique used to draw general conclusions from a set of related studies focused on similar experimental treatments and responses (Glass, 1976). Given sufficiently similar study designs, the size of the treatment effects can be pooled to create a meta-data set, which is then analyzed with appropriate weighting based on the variability and size of each study. This meta-analysis aims to provide decision-makers with a quantitative summary of how much vehicle speed is reduced following installation of a DSFS. Factors such as location, timing, vehicle type, posted speed, safety focus, and others are all considered.

## Methods

### Data collection

Publications reviewed in the Literature Review (above) were assessed for availability of the underlying data on vehicle speeds. Data were entered at two levels:

- the descriptor level, which describes the characteristics of each study (the features which identify how the DSFS was implemented, the hypotheses that were evaluated, and the experimental design)

- the data level, which was the least-aggregated level of data reported either in tabular or graphical format in the publication. For calculations of effect size, at a minimum a study would have to report the mean and standard deviation (SD) of vehicle speeds, as well as the number of vehicles observed, at either two points in time or two locations. The difference in whether studies focused on different timing or location of vehicle speed data collection determines which hypotheses can be tested (Section 3.3.2).

Publications in some cases included more than one study, meaning they tested DSFS at different locations or in combinations with additional treatments such as police cruisers or speed radar enforcement; these are treated as separate studies in the data collection, grouped under the same publication ID. Publications employed many different combinations of descriptors in the study design, and effort was made to enter data from any relevant descriptor that could be expected to appear in multiple studies. Key descriptors included posted speed, safety focus, vehicle type, sign type, and distance from sign the vehicle speeds were measured.

### Calculating Effect Size

Meta-analyses were initially used primarily to assess the odds of a particular outcome, such as the odds a drug treatment would result in a favorable outcome for a patient, and then were expanded to more generally test the size and direction of the outcome across different studies. Both types of tests, assessing odds of a categorical outcome or size of a continuous outcome, are dependent on calculating the effect size. For the meta-analysis of DSFS, the outcomes are vehicle speeds at different points in time. Three sets of hypotheses regarding the effectiveness of DSFS can be tested, and each requires a different calculation of effect size. These hypotheses are discussed in detail in the Literature Review (Section 2.7), and summarized here:

- H1. Activation of the DSFS. The extent the DSFS reduces driver speed is the central question for this analysis. The studies tested activation in three ways:

  - Compare the difference between the upstream and adjacent vehicle speeds before activation to the difference between the upstream and adjacent vehicle speeds during activation of the DSFS. This is the ideal comparison, or the true effect (Cruzado and Donnell 2009).

  - Compare vehicle speeds adjacent to the DSFS before activation and then during activation

  - Compare vehicle speeds upstream (of the DSFS) and adjacent (to the DSFS)

- H2. Activation of the DSFS: downstream. The studies tested the activation effect downstream in five ways:

  - Compare a difference of differences which controls for variations in both the time at which the vehicle speeds (before and during activation) are measured and the location at which the vehicle speeds are measured (upstream and downstream).

  - Compare downstream vehicle speeds during activation to downstream vehicle speeds before activation,

  - Compare downstream vehicle speeds to upstream vehicle speeds during activation,

A’.   Compare a difference of differences which controls for variations in both the time at which the vehicle speeds (before and during activation) are measured and the location at which the vehicle speeds are measured (adjacent and downstream),

C’.   Compare downstream vehicle speed to adjacent speeds during activation.

- H3. Deactivation of the DSFS. The comparisons are similar to those for H2, but now vehicle speeds are compared at the DSFS location after the DSFS has been removed to vehicle speeds at the DSFS while the DSFS was activated, or a normalized difference from after to before or after to during installation.

We calculate the effect size (ES) associated with activation of the DSFS appropriately for each hypothesis. The calculation of effect sizes are based on, at a minimum, measurements of speeds at two locations at the same time, or at two points in time at the same location. For the hypothesis H1B, the effect size is based on the difference between the mean speeds adjacent to the DSFS during the period in which it is active, XDSFS and the sample mean speed adjacent the DSFS before activation XBefore.

The ES is this difference, placed relative to units of the pooled standard deviation:

ES=XDSFS-XBeforeSDPooled

Where the pooled standard deviation for this calculation is defined as follows:

SDPooled=(NDSFS-1)SDDSFSNDSFS+NBefore-2+(NBefore-1)SDBeforeNDSFS+NBefore-2

This effect size calculation is also referred to as the standardized mean difference (SMD, Hedges and Olkin 1985).

For other hypotheses based on the comparison of two groups, such as comparing the effect of DSFS at two locations, adjacent to the DSFS or upstream of the DSFS (Hypothesis H1C, Table 2), the effect size calculation is similar. These normalized ES require a slightly modified calculation, where differences in means rather than means themselves are compared. For example, when the activation effect of the DSFS on vehicle speed is normalized for differences during and before installation, at both the site adjacent to the sign and upstream of the sign, the effect size is calculated as follows:

ES=(XAdjacentDSFS-XAdjacentBefore)-(XUpstreamDSFS-XUpstreamBefore)SDPooled

The pooled standard deviation in this case is computed as follows:

SDPooled=(N1-1)SD1+(N2-1)SD2+(N3-1)SD3+(N4-1)SD4N1+N2+N3+N4-4

The subscripts on each variable identify the relevant time and location at which the variable is measured: 1. Adjacent to the DSFS; 2. Adjacent, Before installation; 3. Upstream of the DSFS; and 4. Upstream of the DSFS, Before installation. Figures 4 through 7 in the Literature Review provide the detailed speed reduction formulas for each of the 13 possible combinations of timing and location of vehicle speeds in response to DSFS installation. The specific calculations follow the two forms above, for either the direct or normalized measurement of effect size.

### Hypothesis testing

This meta-analysis investigates a single continuous outcome (speed of vehicles) across multiple studies, with several moderating variables (e.g., posted speed, safety focus, time of day). The use of multiple moderating variables in a meta-analysis has also been called meta-regression (van Houwelingen, Arends, & Stijnen, 2002). This type of analysis is best conducted using a mixed-effect model, where studies are the random effects, and the moderating variables are the fixed effects. "Random effects" refers to variables in the model which are assumed to be representatives of a larger population (Raudenbush, 2009; Hedges & Vevea, 1998), which in this case are the publications and studies within each publication. The studies examined in this analysis are assumed to be representatives of other possible studies that might have been done. Statistically, random effects are variables which are themselves modeled, with their own error terms. "Fixed effects" refers to predictor variables in the model which are directly measured for each study. When possible, the full model was employed as follows:

yi=β0+β1postedspeed+β2distancefromsign+Ui+ei

where i indexes the study. β0 refers to the overall intercept, while the terms β1 and β2 refer to the coefficients for each of the predictor variables. In many cases the posted speed and distance from sign values were not available, thus this report focuses on overall speed reduction effect sizes.

The error term has multiple parts. The first term Ui represents the random effects, which for this model include the effect of study, nested within publication, as well as safety focus and vehicle type. For example, the study-level error term ui is defined as ui∼N(0,τ2), a normally distributed error term with a mean of zero and variance τ2; this variance is estimated in the model. As a random effect, it represents the variability assumed from the larger population of studies from which the specific studies used have been drawn. The random effects allow the model to fit unbalanced data, which is the case for all the hypotheses tested, and to account for the hierarchical nature of the data, with multiple studies being conducted in some publications. This mixed effect approach (mixing random and fixed effects) allows estimates for how the different levels of the random effects contribute to the overall effect, for instance how passenger cars differ from commercial motor vehicles in their response to DSFS installations.

The remaining error term ei is defined as ei∼N(0,vi), meaning that the residual error is normally distributed with a mean of zero and a variance of vi; this variance vi is directly computed in the effect size calculation. The sampling variance vi becomes important in the model fitting process, giving greater weight to studies which have a lower variability in the response, due to a large sample size, a low standard deviation in the speeds, or a combination of the two.

For each of the 13 hypotheses tested, a similar structure of models was used. Establishing which fixed and random effect variables could be used depended on data availability. At a minimum, the statistical analysis always included a term for study, nested within publication ID. Safety focus and vehicle type were included as random effects in nearly all models. Posted speed and distance from DSFS were available for only a subset of the hypothesis tests.

This analysis allows results to be presented on the overall effectiveness of DSFS in reducing speeds for a particular hypothesis, for example the overall effect of DSFS activation when comparing speeds measured at the DSFS sign to speeds measured at the site of the DSFS sign, prior to sign installation (Hypothesis H1B). In addition to this overall measure of speed reduction, this analysis will further allow reporting of how much the posted speed drives the speed reduction, how much the speed reduction differs for passenger cars versus commercial vehicles, and how much the speed reduction differs for school zones, work zones, rural-to-urban transition zones, or other safety focus areas. The combination of multiple hypotheses tested and multiple moderating variables should assist states in determining where and when to deploy DSFS for maximum effect, and the size of the speed reduction they can expect to observe.

All of the effect size calculations and calculations of sample variability vi were carried out in the meta-analysis package metafor (Viechtbauer, 2010) in the statistical programming language R (R Core Team, 2016). Modified effect size calculations for normalized effect sizes were devised, using the metafor functions as a foundation. Hypothesis testing was carried out using the linear mixed-effect model package lme4 (Bates, Maechler, Bolker, & Walker, 2015). The effect size calculations in metafor have been validated against results provided by the software packages Stata, SAS, and SPSS; the results from these other packages either agreed completely or fell within a margin of error expected when using numerical methods. The results of metafor effect size calculations and analyses have also been validated against textbook examples.

## Results

### Summary of data

The literature review identified 43 publications with data appropriate for quantitative meta-analysis. From these publications, 57 studies were identified, with a total of 204 sites at which the effectiveness of DSFS were evaluated (Table 7). By assessing availability of the appropriate data (mean, s.d., and N) within sites at the appropriate locations, the number of publications, studies, and sites available for testing each of hypothesis can be calculated. This shows that hypothesis H1, the effect of DSFS activation at the site of the sign, is the most commonly studied design, followed by H2, the effect of DSFS activation downstream of the sign. Hypothesis H3, the effect of DSFS following deactivation of the sign, was rarely studied, with only three publications providing sufficient data for meta-analysis. Note that these values differ from the Literature Review vote-count analysis, which only required studies report the significance of the result, not necessarily providing the underlying data for a meta-analysis.

Table 7. Summary of the number of publications, studies, and individual sites with data for each the 13 hypotheses.

| Hypothesis | Publications | Studies | Sites |
| --- | --- | --- | --- |
| H1A | 12 | 22 | 81 |
| H1B | 18 | 36 | 186 |
| H1C | 14 | 28 | 123 |
| H2A | 9 | 18 | 85 |
| H2B | 9 | 18 | 85 |
| H2C | 9 | 18 | 85 |
| H2A' | 9 | 18 | 85 |
| H2C' | 9 | 18 | 85 |
| H3A | 2 | 3 | 8 |
| H3B | 3 | 5 | 8 |
| H3C | 2 | 3 | 4 |
| H3A' | 2 | 3 | 4 |
| H3B' | 3 | 5 | 8 |

The actual number of sample units available for each hypothesis test was greater than the number of sites in some cases, if studies separately investigated different vehicle types, for example. If some sites had insufficient data to carry out the statistical test (for example, lacking standard deviations reported), then the number of sample units was less than the number of sites in some cases. The mean effect size, variance, and number of sample units for each of the thirteen hypotheses is presented in Table 8.

The largest effects are noted for decreases in speed due to activation of the sign, at the site of the DSFS (H1B, 2.99 mph decrease). For the downstream activation effects, the largest mean effect size was for the downstream effect, relative to speeds upstream of the DSFS (H2C, 6.98 mph decrease). Finally, deactivation effects were heterogeneous depending on what hypothesis was tested, but had high effect sizes for the reduction in speed between upstream and adjacent sites, following DSFS removal (H3C, 9.44 mph decrease).

Taken together, these results indicate that DSFS installations were consistently effective in reducing driver speeds, typically by 2-4 mph for passenger cars. Speeds generally increased downstream of DSFS installations, again indicating that the location of the installation sees a clear effect on driver behavior. Following removal of the DSFS, the few studies available indicated that vehicle speeds at the former site of the DSFS remained lower than prior to the installation, giving evidence for a ‘halo effect’ following removal of temporary DSFS installations.

These effect size values in Table 8 are the mean effect sizes calculated at the level of a sample unit. The definition of a sample unit varies by hypothesis, and may include a single site measured at two points in time (e.g., H1B), two sites measured at the same time (H1C), or two sites, each measured at two points in time (H1A and other normalized effect sizes). Some hypotheses could be tested for multiple safety foci and vehicle types across numerous studies. The sections below employ statistical models to account for the differences between publications, studies within publications, vehicle types, and safety foci. Further analysis was carried out, where possible, to address the effect on speed reductions both of the posted speed and of the distance between DSFS and measured speed.

Table 8. Summary of mean effect sizes (ES, in mph) across the 13 hypotheses tested, with variance (Vi) and sample size (N) indicated. Statistical modeling of effect sizes follows below.

| Hypothesis Category | Hypothesis | ES | Vi | N |
| --- | --- | --- | --- | --- |
| H1: Activation | H1A | -2.73 | 1.88 | 72 |
|  | H1B | -2.99 | 0.89 | 249 |
|  | H1C | -2.73 | 2.00 | 49 |
| H2: Downstream | H2A | -1.15 | 1.21 | 35 |
|  | H2B | -2.57 | 0.59 | 162 |
|  | H2C | -6.98 | 0.02 | 4 |
|  | H2A' | 2.65 | 1.06 | 67 |
|  | H2C' | 7.21 | 0.41 | 40 |
| H3: Deactivation | H3A | -0.40 | 0.01 | 2 |
|  | H3B | -0.68 | 0.61 | 20 |
|  | H3C | -9.44 | 0.07 | 4 |
|  | H3A' | -2.60 | 0.01 | 2 |
|  | H3B' | 1.89 | 0.37 | 82 |

### H1: Activation hypothesis

The three versions of the activation hypothesis H1 all rely on a measurement of vehicle speeds at the site of the DSFS, but use different baseline comparisons. The simplest, H1B, compares vehicle speeds at the site of the sign between the time before activation and during activation. H1A calculates a similar value, for the difference between activation timing (before and during), normalized to the same difference upstream of the DSFS. H1C compares speeds at the DSFS to speeds upstream, at the same time. Across all three activation hypothesis tests, significant reductions in speed were observed (see Table 9).

Table 9. Summary of effect sizes for decrease in speed for Hypothesis H1A, normalized activation effect of DSFS, by vehicle type. This table breaks out the overall mean effect sizes in Table 8.

| Hypothesis | Vehicle Type | Effect size | Variance | N |
| --- | --- | --- | --- | --- |
| H1A Activation normalized | Unspecified | -2.73 | 0.28 | 31 |
|  | Passenger | -3.90 | 1.04 | 16 |
|  | Truck | -1.97 | 4.40 | 25 |
| H1B Activation at site of DSFS | Unspecified | -2.69 | 0.14 | 89 |
|  | Passenger | -4.52 | 0.60 | 69 |
|  | Truck | -2.13 | 1.72 | 91 |
| H1C Activation effect, relative to upstream | Unspecified | -11.13 | 0.07 | 6 |
|  | Passenger | -1.70 | 1.59 | 21 |
|  | Truck | -1.42 | 2.92 | 22 |

#### H1A: Activation normalized

Examining the mean effect sizes across vehicle types, without carrying out any statistical modeling, it is clear that the largest decreases in speed were observed for passenger cars. This summary shows that normalized speeds were reduced across all vehicle types, providing strong evidence of the effectiveness of DSFS, even when accounting for the expected reductions in speed between locations upstream and adjacent to the site of the DSFS installation.

Overall, the decrease in speed attributed to DSFS, normalized by upstream location, is 2.84 mph, significant effect (p = 0.004). Of the different grouping variables, vehicle type has the greatest influence, with cars showing an additional 0.05 mph decrease. Trucks showed a smaller effect, with the decrease being smaller by 0.16 mph. When carrying out the statistical analysis of these effect sizes, 72 sites were available. After accounting for the variation attributed to publication, study, and site, partitioning effects by safety focus was not possible for this analysis. Partitioning of the effects by site and vehicle type was possible.

Table 10. Summary of mixed effect model of hypothesis H1A, difference in speeds between DSFS before and at time of activation, normalized to the difference before and at time of activation upstream of the sign.

| Modeled Effect Sizes | Estimate | Std. Error | t value |
| --- | --- | --- | --- |
| Overall Effect | -2.843 | 0.991 | -2.869 |
| Vehicle Type Effects: |  |  |  |
| Unspecified | -2.90 |  |  |
| Car | -2.95 |  |  |
| Truck | -2.68 |  |  |
| Safety Focus Effects: |  |  |  |
| Unspecified | -3.16 |  |  |
| Curved Section | -2.27 |  |  |
| School Zone | -3.21 |  |  |
| Signalized Intersection | -2.90 |  |  |
| Transition Zone | -2.79 |  |  |
| Work Zone | -2.73 |  |  |

#### H1B: Activation at site of DSFS

Calculations of effect sizes for H1B, the decrease in speed at the site of the DSFS, show decreases in speed across all vehicle types and locations. The largest raw effect size was observed for passenger cars, where speeds were on average reduced by -4.52 mph.

The statistical analysis of hypothesis H1B, which takes into account the variation at each level of site, study, and publication, as well as different vehicle types and safety foci, shows a highly significant decrease in speeds of 3.56 mph at the location adjacent to the sign (Table 11; p < 0.001). Within passenger cars, this decrease is even more pronounced with an estimated additional decrease of 0.58 mph. Thus, a 4.14 mph decrease is expected for passenger cars at DSFS locations.

The reductions in speeds for trucks adjacent to DSFS is more modest, with the speed reduction of 2.83 mph.

Table 11. Summary of mixed effect model of hypothesis H1B, difference in speeds between DSFS before and at time of activation.

| Modeled Effect Sizes | Estimate | Std. Error | t value |
| --- | --- | --- | --- |
| Overall Effect | -3.562 | 0.736 | -4.843 |
| Vehicle Type Effects: |  |  |  |
| Unspecified | -3.72 |  |  |
| Car | -4.14 |  |  |
| Truck | -2.83 |  |  |

#### H1C: Activation effect, relative to upstream

The difference between speeds upstream and adjacent to DSFS were surprisingly large, with large decreases in speed detected in several studies which did not differentiate by vehicle type (Hallmark, et al., 2007; Reddy, Datta, Savolainen, & Pinapaka, 2008; Roberts & Smaglik, 2012). The mean upstream-adjacent effect size for DSFS activation across all studies was 2.73 mph.

In the statistical model, this difference in upstream and adjacent speeds overall was quite large, with an 8.62 mph decrease estimated (p = 0.008). Due to the limited sample size, safety focus and vehicle type offsets were not able to be calculated. The large decrease in speeds from upstream to adjacent sites can be attributed in particular to the studies of Hallmark et al. (2007), who studied the effect of DSFS installations at transition zones in rural communities, and Gambatese and Jafarnejad (2015) and Reddy et al. (2008), who studied the effects of DSFS installations in work zones.

Table 12. Summary of mixed effect model of hypothesis H1C, difference in speeds between DSFS upstream and adjacent to the sign.

| Modeled Effect Sizes | Estimate | Std. Error | t value |
| --- | --- | --- | --- |
| Overall Effect | -8.615 | 3.254 | -2.647 |
| Publication ID Effects: |  |  |  |
| Gambatese et al. (2015) | -10.41 |  |  |
| Hallmark et al. (2007) | -15.60 |  |  |
| Reddy et al. (2008) | -13.60 |  |  |
| Roberts et al. (2012) | -2.96 |  |  |
| Tribbett et al. (2000) | -0.50 |  |  |

### H2: Downstream Hypothesis

After drivers move past the DSFS installation, the extent to which speeds increase again is an important point to consider. The set of hypotheses in this category test how speeds change downstream of DSFS installations, compared to sites upstream of the DSFS (H1A, H1C, H2A, H2C) or compared to the sites adjacent to the DSFS installation (H2A', H2C'). These hypotheses also test how the DSFS activation changes speeds over time, using a direct comparison of before to during activation t the downstream site (H2B), or a normalized difference combining both the timing and location of the speed measurement (H2A, H2A').

Comparing speeds between upstream and downstream indicates how much speeds remain diminished, compared to the relatively higher speeds upstream. Comparing speeds between DSFS-adjacent sites and downstream indicates how much speeds rise following the intervention of the DSFS.

Table 13. Summary of mean effect sizes for changes in speed for the five hypotheses under Hypothesis H2, activation effect of DSFS downstream, by vehicle type. This table breaks out the overall mean effect sizes in Table 8.

| Hypothesis | Vehicle Type | Effect size | Variance | N |
| --- | --- | --- | --- | --- |
| H2A Downstream normalized, relative to upstream | Unspecified | -1.21 | 0.37 | 16 |
|  | Passenger | -1.58 | 1.06 | 9 |
|  | Truck | -0.66 | 2.70 | 10 |
| H2B Downstream | Unspecified | -3.07 | 0.19 | 28 |
|  | Passenger | -2.73 | 0.41 | 66 |
|  | Truck | -2.21 | 0.88 | 68 |
| H2C Downstream, relative to upstream | Unspecified | -6.98 | 0.02 | 4 |
|  | Passenger | - | - | - |
|  | Truck | - | - | - |
| H2A’ Downstream normalized, relative to adjacent | Unspecified | 1.11 | 0.36 | 15 |
|  | Passenger | 3.89 | 0.82 | 26 |
|  | Truck | 2.30 | 1.70 | 26 |
| H2C’ Downstream, relative to adjacent | Unspecified | 1.26 | 0.02 | 4 |
|  | Passenger | 8.31 | 0.30 | 18 |
|  | Truck | 7.42 | 0.61 | 18 |

#### H2A: Downstream normalized, relative to upstream

As for the activation hypothesis, a normalized difference in speeds can be calculated to address the downstream effect. Similarly to H1A, where the differences in speeds upstream and adjacent were calculated normalized to the timing of DSFS activation, H2A calculates a difference between the upstream and downstream sites, normalized by the timing of activation. Overall, a 1.15 decrease in speeds was maintained, indicating that drivers did not fully recover to pre-DSFS speeds. Passenger cars maintained the greatest reduction compared to upstream speeds, of 1.58 mph.

In the statistical analysis of this hypothesis shows a modest trend for speed to decrease, when normalized, of only 0.94 mph. This tendency was not statistically significant (p = 0.148). In this case, sufficient variation at the grouping level was only available for safety focus. The three safety foci included had similar decreases in speed, with work zones showing the strongest decrease of 1.36 mph.

Table 14. Summary of mixed effect model of hypothesis H2A.

| Modeled Effect Sizes | Estimate | Std. Error | t value |
| --- | --- | --- | --- |
| Overall Effect | -0.942 | 0.652 | -1.445 |
| Safety Focus Effects: |  |  |  |
| Unspecified | -0.68 |  |  |
| Horizontal curve | -0.78 |  |  |
| Work zone | -1.37 |  |  |

#### H2B: Downstream

Calculations of effect sizes for H2B, the decrease in speed downstream of the DSFS, show decreases in speed across all vehicle types and locations. The largest raw effect size was observed for unspecified vehicle type, with a decrease in 3.07 mph, followed by the decrease in speeds for passenger cars of 2.73 mph.

The statistical analysis of hypothesis H2B shows a large decrease in speeds of 5.44 mph at the downstream of DSFS installations, a trend which was not statistically significant. (p = 0.099). This trend was quite similar across vehicle types, with only slightly larger decreases observed for passenger cars, of 5.55 mph. These results should be interpreted with caution, as very large downstream differences were detected in one study, Fontaine et al. 2001, who studied effects of DSFS installations in work zones in Texas.

Table 15. Summary of mixed effect model of hypothesis H2B, difference in speeds downstream of DSFS sites, before and at time of activation.

| Modeled Effect Sizes | Estimate | Std. Error | t value |
| --- | --- | --- | --- |
| Overall Effect | -5.439 | 3.304 | -1.646 |
| Vehicle Type Effects: |  |  |  |
| Unspecified | -5.44 |  |  |
| Car | -5.55 |  |  |
| Truck | -5.33 |  |  |

#### H2C: Downstream, relative to upstream

The downstream effect of DSFS can be further analyzed by focusing on just the time during the DSFS activation. Comparing speeds detected downstream of the DSFS to speeds upstream of the DSFS is the focus of hypothesis H2C. Here, insufficient studies were conducted to carry out either a breakdown of DSFS effectiveness by vehicle type or employ the full statistical model. However, the overall effect can be assessed as the mean effect size, which was a 6.98 mph decrease.

#### H2A': Downstream normalized, relative to adjacent

As with H2A, which normalized the difference by comparing upstream and downstream speeds, before and during DSFS activation, H2A' examines the speeds adjacent and downstream speeds, for both before and during DSFS activation. Compared to H2A, the difference is now that the reference level is adjacent to the DSFS, therefore the relative speed difference may be expected to be a positive value, as vehicles accelerate following the DSFS.

It is clear that passenger cars accelerate following DSFS, using this metric. There is an increase in speeds of 3.89 mph, which is nearly equal to the decrease detected by the normalized metric of H1A of 3.90 mph.

The statistical analysis of these studies shows an overall speed increase of 2.51 (p = 0.002), with passenger cars contributing a 3.36 mph increase in speeds, which exceeds the 2.95 decrease in speeds for passenger cars estimated by the statistical analysis of H1A.

Table 16. Summary of mixed effect model of hypothesis H2A', the normalized activation effect of DSFS, relative to sites adjacent to the DSFS.

| Modeled Effect Sizes | Estimate | Std. Error | t value |
| --- | --- | --- | --- |
| Overall Effect | 2.513 | 0.807 | 3.113 |
| Vehicle Type Effects: |  |  |  |
| Unspecified | 1.84 |  |  |
| Passenger | 3.36 |  |  |
| Truck | 2.34 |  |  |

#### H2C': Downstream, relative to adjacent

Hypothesis H2C' is similar to H2A', in examining the rebound in speeds downstream of the DSFS, relative to the speeds adjacent to the installation. In contrast to H2A', this hypothesis does not normalize speeds before and during the period of activation. As with H2A', a substantial rebound in speeds was detected, with passenger cars increasing speeds 8.31 mph. Note that the sample size of 40 total studies is substantially fewer than the 72 studies examined for the parallel hypothesis H1C, which examines the difference in speeds upstream and adjacent to DSFS, but many more than the 4 studies available for H2C, comparing upstream to downstream.

Due to sample size limitations, the full statistical model employed in other hypothesis tests could not be employed here. The random effect of safety focus was omitted, with the effects of vehicle type and study within publication retained. The estimated rebound in speeds overall was 4.17 mph (p = 0.136). No statistically significant variation was detected for vehicle types. The largest contribution of the rebound in speeds was from the study of Hajbabai et al. (2011), which evaluated the effect of speed photo-radar enforcement along with DSFS installations in work zones.

Table 17. Summary of mixed effect model of hypothesis H2C'.

| Modeled Effect Sizes | Estimate | Std. Error | t value |
| --- | --- | --- | --- |
| Overall Effect | 4.165 | 2.791 | 1.492 |
| Publication ID Effects: |  |  |  |
| Hajbabai et al. (2011) | 8.22 |  |  |
| Reddy et al. (2008) | 3.40 |  |  |
| Roberts et al. (2012) | 0.87 |  |  |

### H3: Deactivation Hypothesis

DSFS installations are not always permanent. For temporary installations as test cases, for instance in transition zones,

Table 18. Summary of mean changes in speed for the five hypotheses under Hypothesis H3, deactivation effect of DSFS, by vehicle type. This table breaks out the overall mean effect sizes in Table 8.

| Hypothesis | Vehicle Type | Effect size | Variance | N |
| --- | --- | --- | --- | --- |
| H3A Deactivation downstream normalized, relative to before | Unspecified | -0.40 | 0.01 | 2 |
|  | Passenger | - | - | - |
|  | Truck | - | - | - |
| H3B Deactivation, relative to before | Unspecified | -0.01 | 0.05 | 16 |
|  | Passenger | -1.10 | 0.08 | 2 |
|  | Truck | -5.70 | 2.26 | 2 |
| H3C Deactivation, relative to upstream | Unspecified | -9.44 | 0.07 | 4 |
|  | Passenger | - | - | - |
|  | Truck | - | - | - |
| H3A’ Deactivation normalized, relative to during | Unspecified | -2.60 | 0.01 | 2 |
|  | Passenger | - | - | - |
|  | Truck | - | - | - |
| H3B’ Deactivation, relative to during | Unspecified | 2.31 | 0.09 | 48 |
|  | Passenger | 1.83 | 0.22 | 17 |
|  | Truck | 0.78 | 0.73 | 17 |

#### H3A: Deactivation downstream normalized, relative to before

Comparing the speeds upstream and adjacent to the site of the DSFS as in H1C is one direct measure of the effectiveness of DSFS. H3A tests the effectiveness of the DSFS after the sign is removed, by carrying out that comparison both before installation and after removal of the DSFS. This comparison allows examination of how much drivers slow down in anticipation of the DSFS, even after the sign is removed.

No statistical analysis was possible for H3A, as only two sample units were available, from studies conducted in work zones in Arizona (Roberts & Smaglik, 2012). These two studies together shows a modest decrease in speeds, but this cannot be tested for statistical significance.

#### H3B: Deactivation, relative to before

Directly comparing speeds at the site of the DSFS, before installation and after removal, is a simpler test of the ‘halo effect’ of DSFS. In contrast to H3A, this test does not normalize for the difference in location upstream and adjacent, and only compares over time. Four publications provided data with which to test this hypothesis, and showed that there was a tendency of approximately 2 mph that remained after DSFS removal. Given the small sample size, this effect is not statistically significant (p = 0.273). These effect sizes were similar across the four categories of safety foci. The consistency of this halo effect is noteworthy, as it indicates a possible remaining benefit of DSFS installation even following removal.

Table 19. Summary of mixed effect model of hypothesis H3B.

| Modeled Effect Sizes | Estimate | Std. Error | t value |
| --- | --- | --- | --- |
| Overall Effect | -2.077 | 1.895 | -1.096 |
| Safety Focus Effects: |  |  |  |
| Unspecified | -1.89 |  |  |
| Pedestrian area | -2.12 |  |  |
| Transition zone | -2.12 |  |  |
| Work zone | -2.18 |  |  |

#### H3C: Deactivation, relative to upstream

Focusing strictly on the period of time following DSFS removal, H3C examines the difference in speeds between sites upstream and adjacent to where the DSFS would be. The calculated effect size, since not normalized by time, may simply indicate the difference in driving behavior between the upstream and adjacent sites, or may also include a halo effect of the DSFS. In this study, only four sample units were available for this test, from two publications (Hallmark, et al., 2007; Roberts & Smaglik, 2012). These results indicate a trend for speeds to decrease between these sites, but the results were limited by sample size and are not statistically significant (p = 0.332).

Table 20. Summary of mixed effect model of hypothesis H3C.

| Modeled Effect Sizes | Estimate | Std. Error | t value |
| --- | --- | --- | --- |
| Overall Effect | -9.438 | 9.737 | -0.969 |
| Safety Focus Effects: |  |  |  |
| Transition zone | -19.04 |  |  |
| Work zone | 0.16 |  |  |

#### H3A': Deactivation adjacent normalized, relative to during

To overcome the conceptual limitations of hypothesis H3C, it is possible to normalize by the difference upstream and adjacent during the period when the DSFS is installed. The calculated value now is the true difference in speeds following DSFS removal, after correcting for the expected difference when DSFS was installed. In this study, only two sample units were available, relying again on the work zone study of Roberts and Smaglik (2012).

#### H3B': Deactivation, relative to during

The final test of the deactivation hypothesis is the test of how speeds at the site of the DSFS change between the time period when the sign was active and when it was removed. This hypothesis, H3B’, modifies H3B in using the time of DSFS activation as the reference speed, rather than the period before the DSFS was installed. H3B’ can also be thought of as a mirror image to the activation hypothesis H1B, which compares speeds during DSFS activation with speeds before installation.

A modest rebound in speeds was detected, with drivers increasing speeds by 0.96 mph, but this was not statistically significant. While 82 sample units were available for this test, the overall rebound in speeds was close to zero for the five publications examined. This indicates that the speed reductions achieved when the DSFS was installed largely remained in effect even following the removal of the sign.

Table 21. Summary of mixed effect model of hypothesis H3B'.

| Modeled Effect Sizes | Estimate | Std. Error | t value |
| --- | --- | --- | --- |
| Overall Effect | 0.956 | 1.036 | 0.923 |
| Publication ID Effects: |  |  |  |
| Cruzado (2009) | 1.12 |  |  |
| Hallmark (2007) | 0.84 |  |  |
| Kamyab (2002) | 0.91 |  |  |
| Pesti (2011) | 1.11 |  |  |
| Roberts (2012) | 0.80 |  |  |

## Bibliography

Bates, D., Maechler, M., Bolker, B., & Walker, S. (2015). Fitting Linear Mixed-Effects Models Using lme4. Journal of Statistical Software, 67(1), 1-48. doi:10.18637/jss.v067.i01

Bertini, R., Monsere, C., Nolan, C., Bosa, P., & Abou El-Seoud, T. (2006). Field evaluation of Myrtle Creek advanced curve warning system. Washington, DC: Federal Highway Administration.

Bertucci, A. (2006). Sign Legibility Rules of Thumb. Bristol, PA: United States Sign Council. Retrieved June 13, 2016, from http://www.usscfoundation.org/USSCSignLegiRulesThumb.pdf

Bloch, S., & Automobile Club of Southern California. (2007). Comparative study of speed reduction effects of photo-radar and speed display boards. Transportation Research Record, 1640.

Bowie, J. (2003). Efficacy of speed monitoring displays in increasing speed limit compliance in highway work zones (Masters's Thesis). Salt Lake City: Brigham Young University.

Brewer, M., Pestie, G., & Schneider, W. (2006). Improving compliance in work zone speed limits. Transportation Research Record, 67-76.

Bullough, J., Skinner, N., Brons, J., & Rea, M. (2012). Using Lighting And Visual Information to Alter Driver Behavior. Albany, NY: New York State Department of Transportation.

Chang, K., Nolan, M., & Nihan, N. (2004). Radar speed signs on neighborhood streets: An effective traffic calming device? 2004 ITE Annual Meetingn. Lake Buena Vista, FL.

Christ, S., & Abrams, R. (2006). Abrupt onsets cannot be ignored. Psychonomic Bulletin & Review, 13, 875-880.

City of Bellevue Transportation Department. (2009). Stationary radar sign program 2009 report. Bellevue, WA: Author.

City of Englewood, CO. (2014). Recent accomplishments: Traffic safety improvements at Logan and Eastman. Englewood, CO: Author. Retrieved January 3, 2017, from http://www.englewoodgov.org/inside-city-hall/boards-and-commissions/transportation-advisory-committee/recent-accomplishments

Cooper, H., Hedges, L., & Valentine, J. (2009). The Handbook of Research, Synthesis and Meta-analysis (2nd ed.). New York: Russell Safe.

Cruzado, I., & Donnell, E. (2009). Evaluating effectiveness of dynamic speed display signs in transition ones of two-lane, rural highways in Pennsylvania. Transportation Research Record, 1-8.

Donnell, E., & Cruzado, I. (2007). Effectiveness of speed minder in reducing driving speeds on rural highway in Pennsylvania: Literature synthesis. State College, PA: Pennsylvania Transportation Institute.

Drakopoulos, A., & Uprety, S. (2003). I-43 speed warning sign evaluation. Milwaukee, Wisconsin: Marquette University.

Federal Highway Administration. (2012, October 19). Flexibility in Highway Design. Chapter 3. Functional Classification. Retrieved June 8, 2016, from http://www.fhwa.dot.gov/environment/publications/flexibility/ch03.cfm

Federal Highway Administration. (2012). Manual on Uniform Traffic Control Devices: Standard Hihway Signs (2012 Supplement). Washington, DC: Federal Highway Administration. Retrieved June 13, 2016, from http://mutcd.fhwa.dot.gov/shsm_interim/#rs

Federal Highway Administration. (2013). Highway functional classification concepts, criteria and procedures. Washington, DC: Federal Highway Administration.

Federal Motor Carrier Administration. (2014, March 27). Commercial drivers licences: Drivers. (Federal Motor Carrier Safety Administration) Retrieved June 8, 2016, from https://www.fmcsa.dot.gov/registration/commercial-drivers-license/drivers

Fontaine, M. (2008). Innovative traffic control devices for improving safety at rural short-term maintenance work zones. College Station, TX: Texas Transportation Institute.

Fontaine, M., & Carlson, P. (2001). Evaluation of speed displays and rumble strips in rural-maintenance work zones. Transportation Research Record, 1745, 27-38.

Gambatese, J., & Jafarnejad, A. (2015). Evaluation of Radar Speed Display for Mobile Maintenance. Salem, Oregon: Oregon Department of Transportation Research Unit.

Gambatese, J., & Zhang, F. (2014). Safe and effective speed reductions for freeway work zones Phase 2. Salem, OR: Oregon Department of Transportation.

Glass, G. (1976). Primary, secondary, and meta-analysis of research. Educational Researcher, 5(10), pp. 3-8.

Hajbabie, A., Medina, J., Wang, M., Benekohal, R. F., & Chitturi, M. (2011). Sustained and halo effects of various speed reduction treatments in highway work zones. Transportation Research Record, 118-128.

Hallmark, S., & Hawkins, H. K. (2015). Use of DSFS as a Speed Transition Zone Countermeasures in Small, Rural Communities. 2015 IEEE 18th International Conference on Intelligent Transportation Systems (pp. 1448-1454). Canary Islands, Spain: IEEE Computer Society.

Hallmark, S., Hawkins, N., & Smadi, O. (2015). Evaluation of dynamic speed feedback signs on curves: A national demonstration project. McClean, VA: Office of Infrastructure Research and Development, Federal Highway Administration.

Hallmark, S., Knickerbocker, S., & Hawkins, N. (2013). Evaluation of Low Cost Traffic Calming for Rural Communities -- Phase II [Updated]. Ames: Institute for Transportation, Iowa State University.

Hallmark, S., Peterson, E., Fitzsimmons, E., Hawkins, N., Resler, J., & Welch, T. (2007). Evaluationn of gateway and low-cost traffic-calming treatments for major routes in small, rural communities. Ames: Center for Transportation Research and Education, Iowa State University.

Hedges, L., & Vevea, J. (1998). Fixed- and random-effects models in meta-analysis. Psychological Methods, 3, 486-504.

Jeihani, M., Ardeshiri, A., & Naeeni, A. (2012). Evaluating the effectiveness of dynamic speed display signs. Baltimore, MD: Morgan State University National Transportation Center.

Kamyab, A., Andrle, S., & Kroeger, D. (2002). Methods to reduce traffic speed in high pedestrian areas. St. Paul: Minnesota Department of Transportation.

Kloeden, C., McLean, A., Moore, V., & Ponte, G. (1997). Traveling speed and the risk of crash involvement. Adelaide, Australia: NHMRC Road Accident Research Unit, The University of Adelaide.

Knapp, K., & Robinson, F. (2012). The vehicle speed impacts of a dynamic horizontal curve warning sign on low-volume local roadways. St. Paul: Minnesota Department of Transportation.

Koricheva, J., & Gurevitch, J. (2016). Place of Meta-analysis among Other Methods of Research Synthesis. In J. Koricheva, J. Gurevitch, & K. Mengersen (Eds.), Handbook of Meta-analysis in Ecology and Evolution (pp. 3-13). Princeton, NJ: Princeton University Press.

Lee, C., Lee, S., Choi, B., & Oh, Y. (2005). Effectiveness of speed-monitoring displays in speed reduction in school zones. Transportation Research Record, 27-35.

Mattox, J., Sarasua, W., Ogle, J., Eckenrode, R., & Dunning, A. (2007). Development and evaluation of speed-activated sign to reduce speeds in work zones. Transportation Research Record, 3-11.

McCoy, P., & Pesti, G. (2001). Smart work zone technology evaluations: speed monitoring displays and condition-responsive, real-time travel information systems. Lincoln: Mid-America Transportation Center, University of Nebraska-Lincoln.

McCoy, P., Bonneson, J., & Kollbaum, J. (1995). Speed reduction effects of speed monitoring displays with radar in work zones on interstate highways. Transportation Research Record, 65-72.

Medina, J., Benekohal, R., Hajbabaie, A., Wang, M., & Chitturi, M. (2009). Downstream effects of speed photo-radar enforcement and other speed reduction treatments on work zones. Transportation Research Record, 24-33.

Meyer, E. (2000). Evaluation of two strategies for improving safety in highway work zones. Mid-Continent Transportation Symposium 2000 Proceedings (pp. 62-66). Ames: Iowa State University.

National Highway Traffic Safety Administration. (2014). Traffic safety facts 2012 data: Speeding. Washington, D.C.: Author.

National Highway Traffic Safety Administration. (2017, May). Traffic Safety Facts. 2015 Data. Speeding. Retrieved June 13, 2016, from https://crashstats.nhtsa.dot.gov/Api/Public/ViewPublication/812409

NHTSA. (2010). The Economic and Societal Impact of Speeding. Washington, D.C.: National Highway Transportation Safety Administration.

NHTSA. (2017). Traffic Safety Facts: 2015. Washington, D.C.: National Highway Transportation Safety Administration.

Nilsson, G. (2004). Traffic Safety Dimension and the Power Model to describe the Effect of Speed on Safety. Sweden: Lund Institute of Technology.

O'Brien, S., & Simpson, C. (2012). Use of "Your Speed" changeable message signs in school zones: Experience from North Carolina Safe Routes to School Program. Transportation Research Record.

Pesti, G., & McCoy, P. (2011). Long-term effectiveness of speed monitoring displays in work zones on rural interstate highways. Transportation Research Record, 21-30.

R Core Team. (2016). R: A language and environment for statistical computing. Vienna, Austria. Retrieved from https://www.R-project.org/

Raudenbush, S. (2009). Analyzing effect sizes: Random effects models. In H. Cooper, L. Hedges, & J. Valentine, The Handbook of Research Synthesis and Meta-analysis (pp. 295-315). New York: Russell Sage Foundation.

Reddy, V., Datta, T., Savolainen, P., & Pinapaka, S. (2008). Evaluation of innovative safety treatments: A study of the effectiveness of motorist awareness systme in construction work zones. Tallahassee: Florida Department of Transportation.

Richard, C., Campbell, J., Lichty, M., Brown, J., Chrysler, S., Lee, J., . . . & Reagle, G. (2013). Motivations for speeding, Volume II: Findings report. Washington, DC: National Highway Traffic Safety Administration.

Roberts, C., & Smaglik, E. (2012). Driver feedback on monetary penalty and its impact on work zone speed. Transportation Research Record, 27-34.

Robinson, F. (2012). The vehicle speed impacts of dynamic horizontal curve warning sign on low-volume local roadways. St. Paul: Minnesota Department of Transportation.

Saito, M., & Ash, K. (2005). Evaluation of four recent traffic safety initiatives, Volume IV: Increasing speed limit compliance in reduced speed school ones. Salt Lake City, Utah: Utah Department of Transportation Research Division.

Santiago-Chaparro, K., Chitturi, M., Bill, A., & Noyce, D. (2012). Spatial effectiveness of speed feedback signs. Transportation Research Record, 8-15.

Sarasua, W., Ogle, J., & Chowdhury, M. (2006). Better management of speed control in work zones. Columbia, SC: South Carolina Department of Transportation.

Schoenecker, T., Sandberg, W., Sebastian, K., & Soler, P. (n.d.). Long-term effectiveness of dynamic speed monitoring display (DSMD) signs for speed management at speed limit transitions.

Schroeder, P., Kostynuik, L., & Mack, M. (2013). 2011 National survey of speeding attitudes and behaviors. Washington, DC: National Highway Traffic Safety Administration.

South Carolina Department of Transportation. (2006). Traffic Calming Guidelines. Columbia, SC: Soutch Carolina Department of Transportation. Retrieved June 13, 2016, from http://safety.fhwa.dot.gov/speedmgt/ref_mats/fhwasa09028/resources/SCDOT%20Traffic%20calming%20guidelines.pdf

Tefft, B. (2011). Impact speed and a pedestrain's risk of severe injury or death. Washington DC: AAA FTS.

Teng, H., Xu, X., Li, X., Kwigizile, V., & Gibby, A. (2009). Evaluation of speed monitoring displays for work zones in Las Vegas, Nevada. Transportation Research Record, 45-56.

Transportation Research Board. (2010). Highway Capacity Manual (2010). Washington,DC: National Academies of Science.

Tribbett, L., McGowen, P., & Mounce, J. (2000). An evaluation of dynamic curve warning systems in the Sacramento River Canyon. Bozeman, MT: Western Transportation Institute. Retrieved December 13, 2016, from http://www.coe.montana.edu/ce/patm/pubs/files/2000curve.pdf

Tribbett, L., McGowen, P., & Mounce, J. (2000). An evaluation of dynamic curve warning systems in the Sacremento River Canyon. Bozeman, MT: Western Transportation Institute.

Turner, J. (2015, June 10). What's a Work Zone? Retrieved June 8, 2016, from Federal Highway Administration Research and Technology: Public Roads: http://www.fhwa.dot.gov/publications/publicroads/99mayjun/workzone.cfm

Ullman, G., & Rose, E. (2005). Evaluation of dynamic speed display signs. Transportation Research Record, 92-97.

van Houwelingen, H., Arends, L., & Stijnen, T. (2002). Advanced methods in meta-analysis: Multivariate approach and meta-regression. Statistics in Medicine, 21(4), pp. 589-624.

Viechtbauer, W. (2010). Conducting meta-analyses in R with the metafor package. Journal of Statistical Software, 36(3), 1-48. Retrieved from http://www.jstatsoft.org/v36/i03/

Western Transportation Institute. (2003). Greater Yellowstone Rural ITS Project: Work Order II-2C Dynamic Warning VMS Evaluation of Wyoming Site. Bozeman, MT: Western Transportation Institute.

Williamson, M., & Fries, R. (2015). Effectiveness of radar speed signs in a university environment. ite Journal, 44-48.

Note that we could also have written DuringDSFS as DuringAdjacent and similarly BeforeDSFS as BeforeAdjacent (see Figure 4).